HTTP API  Instruction Manual

# HTTP API manual

# HTTP API Specifications

# VER. 1.5

HTTP API  Instruction Manual

# Revision History

| Revision Date | Revisions |  | describe |  | Revised by |
| --- | --- | --- | --- | --- | --- |
| 2023-11-03 | V1.0 |  | Initial Release |  | ENSTER |
| 2023-12-05 | V1.1 |  | Add IO status control |  | ENSTER |
| 2024-01-03 | V1.2 |  | Added support for configuration of fill light brightness mode |  | ENSTER |
| 2024-01-15 | V1.3 |  | 1) Added the Picture option in event notification to carry pictures 2) Add OccurFlag attribute to event notification. True indicates that an alarm is triggered. false means the alarm is over |  | ENSTER |
| 2024-01-16 | V1.4 |  | 1) Event notification adds local time string 2) Added the chapter on setting up the target detection HAPI method |  | ENSTER |
| 2024-03-18 | V1.5 |  | Add video occlusion, face detection, out-of-bounds detection, and area ( perimeter) Detection, smoke detection configuration GET/SET |  | ENSTER |

HTTP API  Instruction Manual

Table of contents

HTTP API manual 	 1

HTTP API Specifications 	 1

VER. 1.5 	 1

1 Introduction 	 7

1.1 HAPI Overview 	 7

1.2            HAPI Adopted HTTP method 	 7

1.3            HAPI Device response message 	 7

1.4 HAPI URL rule 	 8

1.5 HAPI Certification 	 8

1.5.1       Username / password authentication method 	 9

1.5.2       Session ID verification method 	 9

1.6            HAPI Interface sample code 	 9

1.6.1       GET Method plain text password call getuid	 9

1.6.2 PUT Method Call getuid	 9

1.6.3       GET Method Ciphertext Password Call getuid	 10

1.6.4       GET Method Call keep_alive 	 10

1.6.5 PUT Method Call keep_alive	 10

2     Interface Introduction 	 11

2.1            Session ID 	 11

2.1.1 Acquisition uid /HAPI/V1.0/uid/getuid 	 11

2.1.2       Keep alive uid /HAPI/V1.0/uid/keep_alive 	 11

2.2            system message 	 12

2.2.1       Get device information /HAPI/V1.0 /sysinfo/device_info 	 12

2.2.2 Acquisition API Interface List /HAPI/V1.0/sysinfo/functionlist	 13

2.2.3 Obtaining device capability set /HAPI/V1.0/sys info/capability	 16

2.2.4 Acquisition RTSP URL /HAPI/V1.0/sysinfo/rtsp url	 18

2.3            System Control 	 19

2.3.1       Reboot the device /HAPI/V1.0/sysman/reboot 	 19

2.3.2       Restore Factory /HAPI/V1.0/sysman/factory 	 19

2.4            IO State Control 	 20

2.4.1 Acquisition IO Input Status /HAPI/V1.0/io/input/get​	 20

2.4.2 Acquisition IO Output Status /HAPI/V1.0/io/output/get	 20

2.4.3 Settings IO Output Status /HAPI/V1.0/io/output/set​ 	 twenty one

HTTP API  Instruction Manual

2.5            Manual capture 	 twenty two

2.5.1       Manual capture /HAPI/V1.0/snapshot.cgi​	 twenty two

2.6            PTZ control 	 twenty three

2.6.1 PTZ Stop /HAPI/V1.0/ptz_ctrl/stop​ 	 twenty three

2.6.2 PTZ rotation /HAPI/V1.0/ptz_ctrl/move​	 twenty three

2.6.3       PTZ preset point operation /HAPI/V1.0/ptz_ctrl/preset	 twenty four

2.6.4       PTZ zoom operation /HAPI/V1.0/ptz_ctrl/zoom 	 25

2.6.5       PTZ focus operation /HAPI/V1.0/ptz_ctrl/focus	 25

2.6.6       PTZ Aperture Operation /HAPI/V1.0/ptz_ctrl/iris 	 26

2.6.7 PTZ Advanced Function Operations /HAPI/V1.0/ptz_ctrl/advfunction/exec​ 	 27

2.6.8 PTZ advanced function list /HAPI/V1.0/ptz_ctrl/ advfunction/get	 27

2.7            system time 	 28

2.7.1 Obtaining device time and configuration /HAPI/V1.0/systime/gettime​	 28

2.7.2 Set device time and time zone /HAPI/V1.0/systime/settime​ 	 29

2.7.3       Setting up the device NTP Configuration /HAPI/V1.0/systime /setntp	 29

2.8            Lighting Configuration 	 30

2.8.1 Get the supported light switch control mode capability set /HAPI/V1.0/ system/light/ctrlmode/capability 	 30

2.8.2 Get the supported lighting working mode capability set /HAPI/V1.0/ system/light/workmode/capability 	 31

2.8.3       Get light configuration /HAPI/V1.0/system/ light/get	 32

2.8.4       Set the light configuration /HAPI/V1.0/system/light/set	 33

2.9 Image Configuration 	 34

2.9.1       Get image configuration /HAPI/V1.0/system/image/get 	 34

2.9.2       Set image configuration /HAPI/V1.0/system/image/set	 35

2.10 Video Configuration 	 35

2.10.1 Get video encoding capability set /HAPI/V1.0/system/video/capability	 35

2.10.2 Get video encoding configuration /HAPI/V1.0/system/video/get	 37

2.10.3 Set video encoding configuration /HAPI/V1.0/system/video/set	 39

2.11 Audio Configuration 	 40

2.11.1 Get audio configuration encoding capability set /HAPI/V1.0/system/audio/capability	 40

2.11.2 Get audio configuration /HAPI/V1.0/system/audio/get	 41

2.11.3 Set audio configuration /HAPI/V1.0/system/audio/get	 42

2.12          OSD Configuration 	 43

2.12.1 Acquisition OSD config /HAPI/V1.0/syst em/osd/get 	 43

HTTP API  Instruction Manual

2.12.2 Settings OSD config /HAPI/V1.0/ sy stem/osd/set 	 45

2.13 Intelligent Detection Configuration 	 47

2.13.1 Get smart detection capability set /HAPI/V1.0/smart/capability 	 47

2.13.2 Get the target types supported by the target detection (people, vehicles, and non-motor vehicles) algorithm /HAPI/V1.0/smart/objectdetect

/capability 48

2. 13.3 Obtaining target detection (people, vehicles, and non-motor vehicles) algorithm support linkage capability /HAPI/V1.0 /smart /linkage/capability

49

2.13.4 Get the warning sound list /HAPI/V1.0/smart/audiofiles/get	 50

2.13.5 Get motion detection configuration /HAPI/V1.0/motiondetect/get	 51

2.13.6 Set motion detection configuration /HAPI/V1. 0/motiondetect/set	 55

2.13.7 Get object detection configuration /HAPI/V1.0/smart/objectdetect/get	 57

2.13.8 Set the target detection configuration /HAPI/V1.0/smart/objectdetect/set	 61

2.13.9 Get video cover configuration /HAPI/V1.0/smart/videocover/get 	 63

2.13.10 Set video overlay configuration /HAPI/V1.0/smart/videoc over/set	 64

2.13.11 Get face detection configuration /HAPI/V1.0/smart/facedetect /get	 65

2.13.12 Set face detection configuration /HAPI/V1.0/smart/facedetect/set 	 67

2.13.13 Get out-of-bounds detection configuration /HAPI/V1.0/smart/vide ogate/get 	 69

2.13.14 Set cross-border detection configuration 	/HAPI/V1.0/smart/videogate/set 74

2.13.15 Get 	area ( perimeter ) detection configuration /HAPI/V1.0/smart/regionai/get 75

2.13.16 Set regional ( perimeter ) detection configuration /HAPI/V1.0/sma rt/regionai/set	 79

2.13.17 Get license plate recognition configuration /HAPI/V1.0/smart/l pr/get 	 80

2.13.18 Set license plate recognition configuration /HAPI/V1.0/smart/lpr/set	 82

2.13.19 Get smoke and fire detection configuration /HAPI/V1.0/smart/fl ameflumes/get	 84

2.13.20 Set the smoke and fire detection configuration /HAPI/V1.0/smart/flameflumes/set 	 87

3     event 	 89

3.1 Event Workflow 	 89

3.2            Event Subscription 	 90

3.2.1Registration subscription /HAPI/V1.0/ Event/subscription/registration 	 90

3.2.2 Refresh subscription /HAPI/V1.0/Event/ subscription/refresh	 91

3.2.3 Delete subscription /HAPI/V1.0/ Event/subscription/delete	 93

3.3            Event Notification 	 94

3.3.1       Event Notification /HAPI/V1.0 /Event/Notification	 94

HTTP API  Instruction Manual

4 JSON Structured data description 	 97

4.1 Arming Mode​​ 	 97

4.2 Alarm detection and deployment configuration 	 97

4.3 Motion Detection Area Configuration 	 98

4.4 Alarm linkage deployment configuration 	 99

4.5 Alarm sound linkage configuration 	 100

4.6            IO Output linkage configuration 	 100

4.7             Object detection type 	 101

4.8 Target Detection Area Configuration 	 101

4.9 Rectangular Detection Area Configuration 	 102

4.10 Cross-border detection rule configuration 	 102

4.11 Area Detection Rule Configuration 	 104

4.12 Event Notification Js on Block	 105

4.12.1     MotionDetectInfo​	 105

4.12.1     ObjectDetectInfo	 107

HTTP API  Instruction Manual

# 1  introduction

## 1.1 HAPI Overview

HTTP API Referred to as HAPI, using HTTP Short connection mechanism: during one interaction, the client HTTP URL Notification Service  The client needs to request the resource. After receiving the message, the server will URL The specified resource information and request results are as follows Json Format returned.

HAPI support:

1)   Get device capabilities and supported HAPI Method Collection

2)   System control, such as restart, factory reset, etc.

3)   PTZ control, such as rotation, preset point setting, preset point calling, etc.

4)   Obtain and set configurations, such as network, video, audio, storage, alarm, intelligent analysis, PTZ, etc.

5)   Event subscription mechanism

6)   Event alarm reporting

## 1.2 HAPI Adopted HTTP method

HAPI Interface used http Methods include GET/PUT/POST wait.

GET The request message body is empty. The required parameters are in URL The URL is carried in The string starts with ? and carries parameters, which are separated by &. PUT/POST Request parameters are in the message body as json Format portability.

POST Requests need to specify Content-Type:  application/x-www-form-urlencoded.

To facilitate direct debugging in the browser HAPI Interface, most of which is used to obtain information, system control, system control, etc. HAPI Interface support GET Method, in URL Fill in the necessary parameters. Support GET The interface also supports PUT Method, used in the message body json Formats carry more  Item parameters.

Event subscription, refresh, unsubscribe, and notification all use POST method

## 1.3 HAPI Device response message

When the method is GET When HTTP Request in URL The required parameters are carried in the message body, and the message body is empty. When the method is PUT

POST When HTTP The message body usually needs to contain JSON Data. The request message must be sent to the device HTTP/HTTPS port. Equipment JSON The data format is used to respond, usually including the following fields, the meaning of each field is as follows:

ResponseURL : indicates HAPI Request message URL request method

SessionID : getuid Returned interface Session​ ID . If the authentication method is username / password, SessionID Is a null value. ResponseCode : Indicates the system processing result, 0 means success

ResponseString : Succeed indicates success, other text indicates error message

Data : indicates the response result of the request message ( GET ), which is usually JSON Format. URL Same as PUT Request message Data format and GET The data format in the response message is the same. Data The item lists each API If the service

HTTP API  Instruction Manual

data processing failed. is " null ". If The HTTP message method is PUT/POST , in the response message Data is " null ".

{

"Response": {

"ResponseURL": "/HAPI/V1.0/uid/getuid", " SessionID": "15E25D",

"ResponseCode": 0,

"ResponseString": "Succeed", "Data": "null",

} }

## 1.4 HAPI URL rule

/HAPI/V1.0[/Channels/<ID>]/<service-name>/<resource-name>[/<child-resource-name>][/<ID>] ][?

y1>=<value1>[&<key2>=<value2>]] Parameter Description:

| composition | illustrate |
| --- | --- |
| [ ]: | Indicates non-mandatory parameters |
| <> | Indicates a required parameter |
| [/Channels/<ID>] | Channel number, multi-channel devices ID is 0 means the channel number is irrelevant, ID is from 1 Start Table Display channel number (applicable to NVR and multi-channel cameras). This parameter is for monocular cameras Can be omitted. |
| <service-name> | Business type, such as system, media, network, storage, PTZ control, etc. |
| <resource-name> | Resource Name |
| [/<child-resource-name>] | Subresource name, optional |
| [?<key1>=<value1>[&<key2>=<value2> ]] | Start with ? and carry the required parameters. Use & to separate multiple parameters. |

## 1.5 HAPI Certification

Equipment received HAPI When a request is made, the requested API​  URL For requests that fail the legality verification, A failure message should be sent to the requester.

HTTP request carries Session ID Or username / password as authentication information, GET Request in URL Carry in, PUT/POST request Carried in the message body.

HTTP API  Instruction Manual

## 1.5.1 Username / Password Authentication Method

Directly carry username and password information, password supports plain text and 32 Bit MD5 encrypted string:

[username=<username>&password=<password>] The case of the username / password string does not affect the authentication result.

## 1.5.2 Session ID verification method

First, through /HAPI/V1.0/uid/getuid Interface acquisition Session  ID , carrying the username and password information described in the previous section. use HAPI The interfaces need to carry this Session ID .

Session ID After the last successful request 60 seconds timeout, you need to re-request after failure Session ID . have HAPI requests but hopes Session ID It is not invalid, you can use /HAPI/V1.0/uid/ keep_alive Keep your heart beating.

[uid=<uid>]

## 1.6  HAPI Interface sample code

## 1.6.1 GET Method plain text password call getuid

GET / HAPI / V1.0 / uid / getuid ? username = admin & password = 123456

Returns:

{

"Response" : {

,

"ResponseURL" : "/HAPI/V1.0/u id/getuid" "SessionID" : "371C1A 3" ,

"ResponseCode" : 0 ,

"ResponseString" : "Succeed" , "Data" : "null"

} }

## 1.6.2 PUT Method Call getuid

PUT / HAPI / V1.0 / uid / getuid HTTP /1.1

User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache

HTTP API  Instruction Manual

content-type: application/json Accept : */*

Accept-Encoding: gzip, deflate, br Connection: keep-alive

Host : 192.168.1.202 Content - Length : 82

{

" username ": " admin ",

" password ":"e10 adc 3949 ba 59 abbe 56e057f20f883e" }

Returns:

{

" Response " : {

,

" ResponseURL " : " /HAPI/V1.0/uid/getuid "​​​​ " SessionID " : "3729116" ,

" ResponseCode " : 0 ,

" ResponseString " : " Succeed " , " Data " : " null "

} }

## 1.6.3 GET Method Ciphertext Password Call getuid

GET / HAPI /V1.0/ uid / getuid ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e

## 1.6.4 GET Method Call keep_alive​​

GET / HAPI / V1.0 / uid / keep _ alive ? uid = 15E25D

## 1.6.5 PUT Method Call keep_alive​​

PUT / HAPI / V1.0 / uid / keep _ alive HTTP /1.1

User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache

content-type: application/json Accept : */*

Accept-Encoding: gzip, deflate, br Connection: keep-alive

HTTP API  Instruction Manual

Host : 192.168.1.202 Content - Length : 25

{

" uid ":"3 CFABD 6" }

# 2 Interface Introduction

## 2.1 Session ID

## 2.1.1 Acquisition uid / HAPI / V1.0 / uid / getuid

| URL | /HAPI/V1.0/uid/getuid |  |
| --- | --- | --- |
| Description | Obtain uid , used for subsequent API Session |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / uid / getuid ? username = admin & password = e10 adc 3949 ba 59 abb e56e057f20f883e |  |
| URL params ( simple parameters number ) Input Data(PUT) | username/password |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAP I/V1.0/uid/getuid" "SessionID" : "3CC2457" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : "null" } } | , |
| Note |  |  |
| Status |  |  |

## 2.1.2 Keep Alive uid / HAPI / V1.0 / uid / keep _ alive

| URL | /HAPI/V1.0/uid/keep_alive​ |
| --- | --- |
| Description | periodically ( within 60 seconds) to The uid is always valid on the device |

HTTP API  Instruction Manual

| Method | GET/PUT |
| --- | --- |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / uid / keep _ alive ? uid = 3 CFABD 6 |
| URL params ( simple parameters number ) Input Data(PUT) | uid |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/uid/keep_alive " , "SessionID" : "3CC2457" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : "null" } } |
| Note |  |
| Status |  |

## 2.2 System Information

## 2.2.1 Obtaining device information /HAPI/V1.0/sysinfo/device_info​​​​​​​

| URL | /HAPI/V1.0/sysinfo/device_info |  |
| --- | --- | --- |
| Description | Get device information |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / sysinfo / device _ info ? username = admin & password = 12 3456 |  |
| URL params ( simple parameters number ) Input Data(PUT) | username/password or uid |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/sys info/device_info" , "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : { "SN" : "EF000000000000A3" , "device_type" : "YM800S" , "model" : "MC800S" , "ether" : "F0:00:00:00:00:A 3" , "kernelversion" : "Linux 4.9.84 #73 SMP PREEMPT Mon Feb 28 12:36:32 CST 2022 armv7l" "fsversion" : "YM800S_AF_V0 Y EN-RTMP-H5_L V3.2.5.3 build 2023-10-31 15:04:32 " } | , |

12

HTTP API  Instruction Manual

|  | } } |
| --- | --- |
| Note |  |
| Status |  |

## 2.2.2 Acquisition API Interface List /HAPI/V1.0/sysinfo/functionlist​​​​​

| URL | /HAPI/V1.0/sysinfo/functionlist |
| --- | --- |
| Description | Get device support API Interface List |
| Method | GET/PUT |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ sysinfo / functionlist ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters number ) Input Data(PUT) | username/password or uid |
| Success Return Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ sysinfo / functionlist " , " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : [ { " api " : " /HAPI/V1.0[/Channels/ID]/uid/getuid "​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/uid/keep_alive "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/sysinfo/device_info "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/sysinfo/functionlist "​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/sysinfo/capability "​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/sysinfo/rtspurl "​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ sysman / reboot " |

HTTP API  Instruction Manual

|  | }, { " api " : " /HAPI/V1.0[/Channels/ID]/sysman/factory "​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/io/input/get "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/io/output/get "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/io/output/set "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/systime/gettime "​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ systime / settime " }, { " api " : " /HAPI/V1.0[/Channels/ID]/systime/setntp "​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/stop "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/move "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/preset "​​​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ ptz _ ctrl / zoom " }, { " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/focus "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/iris "​​​​​​​​​​ }, { " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/advfunction/exec "​​​​​​​​​​​​ }, { |
| --- | --- |

HTTP API  Instruction Manual

|  | " api " : " /HAPI/V1.0[/Channels/ID]/ptz_ctrl/advfunction/get "​​​​​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / light / ctrlmode / capability " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / light / workmode / capability " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / light / get " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / light / set " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / image / get " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / image / set " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / video / capability " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / video / get " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / video / set " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / audio / capability " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / audio / get " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / audio / set " }, { " api " : " /HAPI/V1.0[/Channels/ID]/system/osd/get "​​​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ system / osd / set " }, |
| --- | --- |

HTTP API  Instruction Manual

|  | { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Smart / capability " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Smart / audiofiles / get " }, { " api " : " /HAPI/V1.0[/Channels/ID]/Smart/objectdetect/capability "​​​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Smart / linkage / capability " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Smart / motiondetect / get " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Smart / motiondetect / set " }, { " api " : " /HAPI/V1.0[/Channels/ID]/Smart/objectdetect/get "​​​​​​​​​​ }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Smart / objectdetect / set " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Event / subscription / regist " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Event / subscription / refresh " }, { " api " : "/ HAPI /V1.0[/ Channels / ID ]/ Event / subscription / delete " } ] } } |
| --- | --- |
| Note | API interface list through json array return |
| Status |  |

## 2.2.3 Obtaining device capability set /HAPI/V1.0/sysinfo/capability​​​​​

| URL | /HAPI/V1.0/sysinfo/capability |
| --- | --- |

HTTP API  Instruction Manual

| Description | Get device capability set |  |
| --- | --- | --- |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / sysinfo / capability ? username = admin & password = e10 adc 3949b a59 abbe 56e057f20f883e |  |
| URL params ( simple parameters number ) Input Data(PUT) | username/password or uid |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/sysinfo/capability " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : [ { "caps" : "gpio_input" }, | , |
|  | { "caps" : "dzoomsetting" } ] } } |  |
| Note |  |  |
| Status |  |  |

List of capability set strings that need attention, such as lighting, intelligence, and coding

| ptz_control | With PTZ control capability |
| --- | --- |
| audio_action | Support alarm linkage sound output |
| ledtype_set | Support setting fill light mode: intelligent dual light / pure infrared / pure white light |
| ir_led_panel | Pure infrared light board, fill light mode does not support switching to white light / dual light mode |
| white_led_panel | Pure white light board, fill light mode does not support switching to white light / dual light mode |
| double_led_panel | Intelligent dual light board |
| ircut_setting | Support settings IRCUT Mode (Soft Light Auto, External, Timed) |
| ircut_ledmanualswitch | IRCUT Mode Addition 2 Type: Manual light always on, manual light always off ( specifically which light is determined by the fill light mode Decide ) |
| ircut_leddelay | Support setting the sensitivity of soft light switch |
| AlarmVG | Wire-trapping detection |
| AlarmRegionAI | Area Detection |
| VideoPD | Vehicle and person detection |
| face_detect | Face Detection |

# HTTP API  Instruction Manual

| face_recognize | Face Recognition |
| --- | --- |
| VehicleCar | Vehicle shape detection - car |
| VehicleMoto | Vehicle shape detection - Motorcycle |
| VehicleElectricbicycle | Vehicle Type Detection - Motorcycle |
| VehicleBicycle | Vehicle Type Detection - Bicycle |
| LPRecognition | License Plate Recognition |
| Fire | Flame detection |
| three_video | Support video third stream |
| twolens_stitch | Binocular stitching camera |

## 2.2.4 Acquisition RTSP URL / HAPI / V1.0 / sysinfo / rtspurl

| URL | /HAPI/V1.0/sysinfo/rtspurl |  |
| --- | --- | --- |
| Description | Obtain RTSP URL |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / sysinfo / rtspurl ? username = admin & password = e10 adc 394 9 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/sysinfo/rtspurl " , "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : { "ch0_main" : "rtsp://192.1 68.1.202:554/stream0" "ch0_sub" : "rtsp:// 192.168 .1.202 :554/stream1" } } } | , |
| Note |  |  |
| Status |  |  |

# HTTP API  Instruction Manual

## 2.3 System Control

## 2.3.1 Restart the device /HAPI/V1.0/sysman/reboot​​​​​

| URL | /HAPI/V1.0/sysman/reboot |  |
| --- | --- | --- |
| Description | Reboot the device |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / sysman / reboot ? username = admin & password = e10 adc 3 94 9 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/ V1.0/sysman/reboot" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : "null" } } | , |
| Note |  |  |
| Status |  |  |

## 2.3.2 Factory Reset / HAPI / V1.0 / sysman / factory

| URL | /HAPI/V1.0/sysman/factory |  |
| --- | --- | --- |
| Description | Restore Factory |  |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/sysman / factory ? username = admin & password = e10 adc 394​ 9 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Method | GET/PUT |  |
| Input Data | none |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/V 1.0/sysman/factory" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : "null" } | , |

# HTTP API  Instruction Manual

|  | } |
| --- | --- |
| Note |  |
| Status |  |

## 2.4 IO State Control

## 2.4.1 Acquisition IO Input Status / HAPI / V1.0 / io / input / get

| URL | /HAPI/V1.0/io/input/get |  |
| --- | --- | --- |
| Description | Obtain IO input status |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/io/input/get?chn=1&username=admin&password=e10adc3949ba59abbe5​​​​​​​​​​​​​​​​​​​​​​​​​ 6e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | chn : IO Channel number, from 1 Start, according to IPC Actual Configuration username/password or uid |  |
| Success Return Data | { " Response " : { , " ResponseURL " : " /HAPI/V1.0/io/input/get "​​​​​​ " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : 0 } } |  |
| Note | the IO channel number does not exist, an error is returned: { " Response " : { , " ResponseURL " : " /HAPI/V1.0/io/input/get "​​​​​​ " SessionID " : "" , " ResponseCode " : -1 , " ResponseString " : " params error " , " Data " : " null " } } |  |
| Status |  |  |

## 2.4.2 Acquisition IO Output Status /HAPI/V1.0/io/output/get​​​​​​​

| URL | /HAPI/V1.0/io/output/get |
| --- | --- |
| Description | Obtain IO output status |

# HTTP API  Instruction Manual

| Method | GET/PUT |  |
| --- | --- | --- |
| Sample | http://192.168.1.202/HAPI/V1.0/io/output/get?chn=1&username=admin&password=e10adc3949ba59abbe56e057f20f883e​​​​​​​​​​​​​​​​​​​​​​​​​ |  |
| URL params ( simple parameters ) Input Data(PUT) | chn : IO Channel number, from 1 Start, according to IPC Actual Configuration username/password or uid |  |
| Success Return Data | { " Response " : { " ResponseURL " : " /HAPI/V1.0/io/output/get "​​​​​​ " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : 1 } } | , |
| Note | the IO channel number does not exist, an error is returned: { " Response " : { " ResponseURL " : " /HAPI/V1.0/io/output/get "​​​​​​ " SessionID " : "" , " ResponseCode " : -1 , " ResponseString " : " params error " , " Data " : " null " } } | , |
| Status |  |  |

## 2.4.3 Settings IO Output Status / HAPI / V1.0 / io / output / set

| URL | /HAPI/V1.0/io/output/set |  |  |
| --- | --- | --- | --- |
| Description | Obtain IO output status |  |  |
| Method | GET/PUT |  |  |
| Sample | http://192.168.1.202/HAPI/V1.0/io/output/set?chn=1&status=0&username=admin&password=e10adc3949​​​​​​​​​​​​​​​​​​​​​​​ ba 59 abbe 56e057f20f883e |  |  |
| URL params ( simple parameters ) Input Data(PUT) | chn : IO Channel number, from 1 Start, according to IPC Actual Configuration username/password or uid |  |  |
| Success Return Data | { " Response " : { "ResponseURL" : " SessionID " : "" "ResponseCode" : | "/ HAPI /V1.0/ io / output / set " , 0 , | , |

# HTTP API  Instruction Manual

|  | " ResponseString " : " Succeed " , "Data" : 0 } } |  |
| --- | --- | --- |
| Note | the IO channel number does not exist, an error is returned: { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ io / output / set " " SessionID " : "" , " ResponseCode " : -1 , " ResponseString " : " params error " , " Data " : " null " } } | , |
| Status |  |  |

# 2.5 Manual capture

## 2.5.1 Manual capture /HAPI/V1.0/snapshot.cgi​​​​​

| URL | /HAPI/V1.0/snapshot.cgi |
| --- | --- |
| Description | Get the image |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ snapshot . cgi ? stream = 1& username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e http :// 192.168.1.202 / HAPI /V1.0/ snapshot . cgi ? stream = 0 & username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | Stream : Capture stream number, stream=0 means capture sub-stream, stream=1 means capture sub-stream Indicates capturing the main stream; username/password or uid |
| Method | GET/PUT |
| Input Data | none |
| Success Return Data | Jpg picture |
| Note |  |
| Status |  |

HTTP API  Instruction Manual

## 2.6 PTZ control

## 2.6.1 PTZ Stop /HAPI/V1.0/ptz_ctrl/stop​​​​​​​

| URL | /HAPI/V1.0/ptz_ctrl/stop |  |
| --- | --- | --- |
| Description | Stop PTZ control |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / ptz _ ctrl / stop ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/ V1.0/ptz_ctrl/stop" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : "null" } } | , |
| Note |  |  |
| Status |  |  |

## 2.6.2 PTZ rotation / HAPI / V1.0 / ptz _ ctrl / move

| URL | /HAPI/V1.0/ptz_ctrl/move |  |
| --- | --- | --- |
| Description | Control the pan/tilt rotation direction, support 8 Direction Control |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/ptz_ctrl/move?direction=left&autostop=500&speed=5&username=admin&p​​​​​​​​​​​​​​​​​​​​​​​​ assword =e10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | direction : PTZ moving direction, value range: "left", "right", " up", "down", "left_up", "right_up", "left_down", "right_down" ; speed : PTZ speed, value range 1-10 ; autostop : automatically stops when the control reaches the specified milliseconds, blocking call, greater than 1000 Milliseconds The value will be 1000 Milliseconds is automatically stopped. Leave it blank or 0 When it is not automatically stopped username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : " /HAPI/V1.0/ptz_ctrl/move "​​​​​​ " SessionID " : "" , | , |

HTTP API  Instruction Manual

|  | " ResponseCode " : 0 , " ResponseString " : " Succeed " "Data" : { " direction " : " left " , " speed " : 5 , " autostop " : 500 } } } | , |
| --- | --- | --- |
| Note |  |  |
| Status |  |  |

## 2.6.3 PTZ preset point operation / HAPI / V1.0 / ptz _ ctrl / preset

| URL | /HAPI/V1.0/ptz_ctrl/p reset |  |
| --- | --- | --- |
| Description | Control the pan/tilt rotation direction, support 8 Direction Control |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / ptz _ ctrl / preset ? method = set & presetno = 1 & username = admi n& password =e10 adc 3949 ba 59 abbe 56e057f20f883e http :// 192.168.1.202 / HAPI / V1.0 / ptz _ ctrl / preset ? method = call & presetno = 2 & uid = 3 CFABD 6 http :// 192.168.1.202 / HAPI / V1.0 / ptz _ ctrl / preset ? method = delete & presetno = 3 & uid = 3 CFABD 6 |  |
| URL params ( simple parameters ) Input Data(PUT) | method : Preset point operation method, value range: "set", "call", " delete" ; presetno : Preset point number, value range 1-255 ;​ username/password or uid |  |
| Success Response Data | { "Response" : { , "ResponseURL" : "/HAPI/ V1.0/ptz_ctrl/move" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , " Data" : { "direction" : "left" , "speed" : 5 } } } |  |
| Note |  |  |
| Status |  |  |

HTTP API  Instruction Manual

## 2.6.4 PTZ zoom operation / HAPI / V1.0 / ptz _ ctrl / zoom

| URL | /HAPI/V1.0/ptz_ctrl/ zoom |  |
| --- | --- | --- |
| Description | Control pan/tilt zoom |  |
| Method | GET/PUT |  |
| Sample | enlarge: http://192.168.1.202/HAPI/V1.0/ptz_ctrl/zoom?direction=in&autostop=500&username=admin&password=e1​​​​​​​​​​​​​​​​​​​​​​​ 0 adc 3949 ba 59 abbe 56e057f20f883e Zoom out: http://192.168.1.202/HAPI/V1.0/ptz_ctrl/zoom?direction=out&autostop=100&username=admin&password=​​​​​​​​​​​​​​​​​​​​​​​ e10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | Direction : Zoom direction, value range: "i n", "out" ; autostop : automatically stops when the control reaches the specified milliseconds, blocking call, greater than 1000 Milliseconds The value will be 1000 Milliseconds is automatically stopped. Leave it blank or 0 When it is not automatically stopped username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : " / HAPI /V1.0/ ptz_ctrl / zoom "​ " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " direction " : " in " , " autostop " : 500 } } } | , |
| Note | Response Carry control direction. |  |
| Status |  |  |

## 2.6.5 PTZ focus operation / HAPI / V1.0 / ptz _ ctrl / focus

| URL | /HAPI/V1.0/ptz_ctrl/focus​ |
| --- | --- |
| Description | Control pan/tilt focus |
| Method | GET/PUT |
| Sample | Focusing Nearby: http://192.168.1.202/HAPI/V1.0/ptz_ctrl/focus?direction=near&autostop=100&username=admin&password=​​​​​​​​​​​​​​​​​​​​​​​ e10 adc 3949 ba 59 abbe 56e057f20f883e Focusing on the distance: http://192.168.1.202/HAPI/V1.0/ptz_ctrl/focus?direction=far&autostop=100&username=admin&password=e​​​​​​​​​​​​​​​​​​​​​​​ |

HTTP API  Instruction Manual

|  | 10 adc 3949 ba 59 abbe 56e057f20f883 e |  |
| --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | Direction : Focus direction, value range: "near ", "far" ; autostop : automatically stops when the control reaches the specified milliseconds, blocking call, greater than 1000 Milliseconds The value will be 1000 Milliseconds is automatically stopped. Leave it blank or 0 It means no automatic stop; username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : " /HAPI/V1.0/ptz_ctrl/focus "​​​​​​ " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " direction " : " near " , " autostop " : 100 } } } | , |
| Note | Response Carry control direction. |  |
| Status |  |  |

## 2.6.6 PTZ Aperture Operation / HAPI / V1.0 / ptz _ ctrl / iris

| URL | /HAPI/V1.0/ptz_ctrl/iris​ |  |
| --- | --- | --- |
| Description | Control PTZ aperture |  |
| Method | GET/PUT |  |
| Sample | Aperture Increase: http://192.168.1.202/HAPI/V1.0/ptz_ctrl/iris?direction=open&autostop=100&username=admin&password=e​​​​​​​​​​​​​​​​​​​​​​​ 10 adc 3949 ba 59 abbe 56e057f20f883e Stop down: http://192.168.1.202/HAPI/V1.0/ptz_ctrl/iris?direction=close&autostop=100&username=admin&password=e​​​​​​​​​​​​​​​​​​​​​​​ 10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | direction : Aperture control direction, value range: "open", "close" ; autostop : automatically stops when the control reaches the specified milliseconds, blocking call, greater than 1000 Milliseconds The value will be 1000 Milliseconds is automatically stopped. Leave it blank or 0 It means no automatic stop; username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : " /HAPI/V1.0/ptz_ctrl/iris "​​​​​​ " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , | , |

HTTP API  Instruction Manual

|  | "Data" : { " direction " : " open " , " autostop " : 100 } } } |
| --- | --- |
| Note | Response Carry control direction. |
| Status |  |

## 2.6.7 PTZ Advanced Function Operations /HAPI/V1.0/ptz_ctrl/advfunction/exec​​​​​​​​​

| URL | /HAPI/V1.0/ptz_ctrl/advfunction/exec |  |
| --- | --- | --- |
| Description | Execute advanced PTZ functions |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / ptz _ ctrl / advfunction / exec ? functionname = PTZReset & userna me = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | functionname : advanced function name. The value range needs to be determined according to the device configuration. username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/ptz_ctrl/ advfunction/exec" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , " Data" : { "functionname" : "PTZReset" } } } | , |
| Note | The device can be configured with a series of PTZ advanced functions, each of which performs the setting or calling of one or more preset points. |  |
| Status |  |  |

## 2.6.8 PTZ advanced function list / HAPI / V1.0 / ptz _ ctrl / advfunction / get

| URL | /HAPI/V1.0/ptz_ctrl/adv function/get |
| --- | --- |
| Description | Get the device gimbal advanced function list |
| Method | GET/PUT |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / ptz _ ctrl / advfunction / get ? username = admin & password = e10 |

HTTP API  Instruction Manual

|  | adc 3949 ba 59 abbe 56e057f20f883e |  |  |  |
| --- | --- | --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |  |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/ptz_ct rl/advfunction/get" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : [ { "functionname" : "GuardPositionOff" }, |  |  | , |
|  | { "functionname" }, { "functionname" } ] } } | : : | "TrackAutoOff" "TrackAutoOn" |  |
| Note | The device can be configured with a series of PTZ advanced functions, each of which performs the setting or calling of one or more preset points. API To obtain A list of more advanced features that have been configured. |  |  |  |
| Status |  |  |  |  |

## 2.7 System Time

## 2.7.1 Obtaining device time and configuration /HAPI/V1.0/systime/gettime​​​​​

| URL | /HAPI/V1.0/systime/gettime |
| --- | --- |
| Description | Get the current time and time configuration of the device |
| Method | GET/PUT |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ systime / gettime ? username = admin & password = e10 adc 394 9 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |

HTTP API  Instruction Manual

| Success Return Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/systime/gettime " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : { | , |
| --- | --- | --- |
|  | "timeMode" : "MANUAL" , "timeZone" : 1200 , "nowtime" : "2023-11-06 20:13:4 6" } } } |  |
| Note | timeMode: time synchronization mode, Manual , P2P , NTP timeZone : time zone, integer , ( time zone + 12 ) * 60 , for example, Eastern 8 District time Zone Value is 1200 nowtime : The local time string of the device, formatted in the order of year, month, day, hour, minute, and second as "%04d-%02d-%02d %02d:%02d:%02d" |  |
| Status |  |  |

## 2.7.2 Set device time and time zone / HAPI / V1.0 / systime / settime

| URL | /HAPI/V1.0/systime/settime |
| --- | --- |
| Description | Set device time |
| Method | GET/PUT |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / systime / settime ? localtime = 202311062028 40 & timeZone = 1 260& username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | timeZone : time zone, integer , ( time zone + 12 ) * 60 , for example, Eastern 8 District timeZone Value is 1200 , Can be left blank. If not filled in, the device time zone will not be changed. localtime : set up Place local hour between Character symbol string , according to According to Year moon day hour Minutes and seconds Sequential Grid Mode change for "%04d%02d%02d%02d%02d%02d" , can be left blank, and the device time will not be changed if it is not filled in; username/password or uid |
| Success Return Data | See also 2.7.1GET Returns data. |
| Note | " |
| Status |  |

## 2.7.3 Setting up the device NTP Configuration /HAPI/V1.0/systime/setntp​​​​​

| URL | /HAPI/V1.0/systime/ setntp |
| --- | --- |
| Description | Setting up the device NTP Configuration |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/systime/setntp?serverIP=ipvs.icamra.com&username=admi​​​​​​​​​​​​​​​​​​ n& password =e10 adc 3949 ba 59 abbe 56e057f20f883e |

29

HTTP API  Instruction Manual

| URL params ( simple parameters ) Input Data(PUT) | NTP serverIP/server Port/refreshInterval , username/password or uid |
| --- | --- |
| Success Return Data | See also 2.7.1GET Returns data. |
| Note | serverPort Can be omitted, the default is 123. refreshInterval Can be omitted, the default is 60 seconds) |
| Status |  |

## 2.8  Lighting Configuration

2.8.1 Acquisition Pick branch hold of lamp Light open close control system mold Mode able force set /HAPI /V1.0/

system / light / ctrlmode / capability

| URL | ## /HAPI/V1.0/system/light/ctrlmode/capability |
| --- | --- |
| Description | Get a list of supported light switch control modes |
| Method | GET/PUT |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / system / light / ctrlmode / capability ? username = admin & passw ord =e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/system/light/ctrl mode/capability" , "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : [ { "mode" : 0 }, { "mode" : 1 }, { "mode" : 2 }, { "mode" : 3 } ] |

HTTP API  Instruction Manual

|  | } } |
| --- | --- |
| Note | Light switch control mode (also called IRCUT control mode, day and night switching mode ), used to configure the light on , switch IRCUT , and images that turn from color to gray Control mode, the value range is: 0 : Active mode / soft photosensitivity automatic control mode , ISP automatic judgment SENSOR Gain to control 1 : Day and night mode, controlled according to time period 2: Passive mode / hard photosensitivity external control mode, the light board turns on the light according to the hardware signal given by its own photoresistor, and notifies the camera through high and low levels. The camera switches according to the signal IRCUT and image color to gray 3 : Manual mode, manually switched by the caller, mainly used for interface testing IRCUT Function 4 : Reverse passive mode, deprecated 5 : Hard photosensitivity automatic control mode, the module is based on the photoresistor hardware adc Data to control 6 : Manual light always on 7 : Manual light always off |
| Status |  |

2.8.2 Obtain Pick branch hold of lamp Light work do mold Mode able force set /HAPI/V1.0/​

system / light / workmode / capability

| URL | ## /HAPI/V1.0/system/light/workmode/capability |  |
| --- | --- | --- |
| Description | Get a list of supported light switch control modes |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ system / light / workmode / capability ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f8 83e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/system/light/workmode/capability " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : [ { "mode" : 0 }, { "mode" : 1 }, { | , |

HTTP API  Instruction Manual

|  | "mode" : 2 } ] } } |
| --- | --- |
| Note | Lighting working mode, used to configure the working mode of the fill light when the light is turned on. The value range is: 0 : Pure infrared, only turn on the infrared light when the light is needed 1 : Pure white light, only turn on the white light when it is necessary to turn on the light 2 : Intelligent dual light, under the condition that the light needs to be turned on, according to the image intelligent detection results, when there is a detection event, turn on the white light and keep the color image, and when there is no detection Turn on infrared light when event occurs |
| Status |  |

## 2.8.3 Get light configuration / HAPI / V1.0 / system / light / get

| URL | /HAPI/V1.0/system/light/get |  |
| --- | --- | --- |
| Description | Get Lighting Configuration |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ system / light / get ? username = admin & password = e10 adc 394 9 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/system/light/get " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : { "led_ctrl_mode" : 2 , "led_work_mode" : 0 , "light_open_brightness" : 3 , "light_off_sensitivity" : 50 , " led_brightness_mode" : 0 , "led_brightness_value" : 1 00 , "night_starttime" : "18:00:00" , "night_endtime" : "08:00:00" } } } | , |
| Note | led_ctrl_mode: light switch control mode, the value range is shown in 2.8.1 |  |

HTTP API  Instruction Manual

|  | led_work_mode: Lighting working mode, the value range is shown in 2.8.2 ; light_open_brightness: Turn on the ambient light brightness, value range 0-17 , the smaller the value, the lower the ambient brightness, the light will be turned on; light_off_sensitivity: Light off sensitivity, value range 0-100 , used to control the sensitivity of turning off the light after the brightness changes after turning on the light ; led_brightness_mode : fill light brightness mode: value range 0-2 ( 0 : Automatic; 1 : Manual; 2 : Low Power Consumption) led_brightness_value: fill light brightness, value range 0-100 ; night_starttime : night​​ Late start between, For configuration of day and night hour Timer to turn on the light , formatted in the order of hours, minutes and seconds "%02d:%02d:%02d" ; night_endtime : night​​ Night Finish hour between ,use Yu An According to the configuration of day and night hour Room to decide hour Turn off the lights ,according to According to Hours, minutes, and seconds are formatted in this order: "%02d:%02d:%02d" ; |
| --- | --- |
| Status |  |

## 2.8.4 Set lighting configuration / HAPI / V1.0 / system / light / set

| URL | /HAPI/V1.0/system/light/set |
| --- | --- |
| Description | Set up lighting configuration |
| Method | GET/PUT |
| Sample | Set the lighting control mode to day and night mode: http :// 192.168.1.202 / HAPI / V1.0 / system / light / set ? led _ ctrl _ mode = 1& night _ endtime = 093000 & night _ starttime = 153500& username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f 883e Set the fill light working mode to intelligent dual light: http :// 192.168.1.202 / HAPI / V1.0 / system / light / set ? led _ work _ mode = 2 & username = admin & pa ssword =e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | Fill in one or more of the following parameters. An error will be returned if a parameter is missing or the range is incorrect: led_ctrl_mode: light switch control mode, the value range is shown in 2.8.1 led_work_mode: Lighting working mode, the value range is shown in 2.8.2 ; light_open_brightness: Turn on the ambient light brightness, value range 0-17 , the smaller the value, the lower the ambient brightness, the light will be turned on; light_off_sensitivity: Light off sensitivity, value range 0-100 , used to control the sensitivity of turning off the light after the brightness changes after turning on the light ; led_brightness_mode : fill light brightness mode: value range 0-2 ( 0 : Automatic; 1 : Manual; 2 : Low Power Consumption) led_brightness_value: fill light brightness, value range 0-100 ; night_starttime : night​​ Late start between, For configuration of day and night hour Timer to turn on the light , formatted in the order of hours, minutes and seconds "%02d:%02d%:02d" ; night_endtime : night​​ Night Finish hour between ,use Yu An According to the configuration of day and night hour Room to decide hour Turn off the lights ,according to According to Hours, minutes, and seconds are formatted in this order: "%02d:%02d:%02d" ; username/password or uid |
| Success Response Data | See also 2.8.3GET Returns data. |
| Note | The returned data carries the current lighting configuration data |
| Status |  |

HTTP API  Instruction Manual

## 2.9  Image Configuration

## 2.9.1 Get image configuration / HAPI / V1.0 / system / image / get

| URL | /HAPI/V1.0/system/image/get |  |
| --- | --- | --- |
| Description | Get the current configuration of the image |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / system / image / get ? username = admin & password = e10 adc 39 49 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/system/image/get " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succee d" , "Data" : { "brightness" : 128 , "contrast" : 128 , "saturation" : 128 , "sharpness" : 128 , "tvsystem" : 1 , "hflip" : 0 , "vflip" : 0 , "rotate" : 0 } } } | , |
| Note | brightness : brightness, 0-255 contrast : contrast, 0-255 saturation : saturation, 0-255 sharpness : 0-255 tvsystem ：Video system 0: NTSC (60HZ) , 1: PAL (50HZ) hflip : horizontal flip vflip : vertical flip rotate : Corridor mode (rotation 90 (supported by some cameras) |  |
| Status |  |  |

HTTP API  Instruction Manual

## 2.9.2 Setting image configuration / HAPI / V1.0 / system / image / set

| URL | /HAPI/V1.0/system/image/set |
| --- | --- |
| Description | Setting the image configuration |
| Method | GET/PUT |
| Sample | To set the brightness: http :// 192.168.1.202 / HAPI / V1.0 / system / image / set ? brightness = 129 & username = admin & pas sword =e10 adc 3949 ba 59 abbe 56e057f20f883e Set the video format: http :// 192.168.1.202 / HAPI / V1. 0 / system / image / set ? tvsystem = 1 & username = admin & password rd =e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | Fill in one or more of the following parameters. An error will be returned if a parameter is missing or the range is incorrect: brightness : brightness, 0-255 contrast : contrast, 0-255 saturation : saturation, 0-255 sharpness : 0-255 tvsystem ：Video system 0: NTSC (60HZ) , 1: PAL (50HZ) hflip : horizontal flip vflip : vertical flip rotate : Corridor mode (rotation 90 degrees, supported by some cameras) ; username/password or uid |
| Success Response Data | See also 2.9.1 GET Returns data. |
| Note | The returned data carries the current image configuration data |
| Status |  |

## 2.10 Video Configuration

## 2.10.1 Get video coding capability set / HAPI / V1.0 / system / video / capability

| URL | /HAPI/V1.0/system/video/capability |
| --- | --- |
| Description | Get the supported video encoding capability set |
| Method | GET/PUT |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ system / video / capability ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |
| Success Response Data | { "Response" : { |

HTTP API  Instruction Manual

|  | "ResponseURL" : "/HAPI/V1.0/system/video/capability " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : [ { "codec_name" : "H265+" , "res_name" : "3840X2160" , "stream_type" : 0 , "def_bitrate" : 6000 , "min_bitrate" : 512 , "max_bitrate" : 12288 , "def_framerate" : 20 , "min_framerate" : 5 , "max_framerate" : 20 , "def_config" : 0 }, ………… { "codec_name" : "H264" , "res_name" : "CIF" , "stream_type" : 2 , "def_bitrate" : 150 , "min_bitrate" : 64 , "max_bitrate" : 2048 , "def_framerate" : 10 , "min_framerate" : 5 , "max_framerate" : 10 , "def_config" : 0 } ] } } | , |
| --- | --- | --- |
| Note | The list of supported video codecs is as follows json Array returned. Meaning of array members : codec_name : encoding name, value is H264 , H265 , H265+ , MJPEG wait res_name : resolution name, usually in terms of resolution width X high means, for example 2560X1440 , some regular resolutions 1080P/CIF Direct representation of strings stream_type : stream number, 0 Main stream, 1 For sub-stream, 2 For three streams , def_bitrate : The default bitrate when this resolution is selected min_bitrate : The minimum bitrate supported by this resolution max_bitrate : The maximum bitrate supported by this resolution def_framerate : The default frame rate when this resolution is selected min_framerate : The minimum frame rate supported by this resolution |  |

HTTP API  Instruction Manual

|  | max_framerate : The maximum frame rate supported by this resolution def_config : Whether this resolution is the default configuration of the camera |
| --- | --- |
| Status |  |

## 2.10.2 Get video encoding configuration / HAPI /V1.0/ system / video / get

| URL | /HAPI/V1.0/system/video/get |  |
| --- | --- | --- |
| Description | Get video encoding configuration |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI /V1.0/ system / video / get ? username = admin & password = e10 adc 39 49 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ system / video / get " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , " stream " : [ { " enable " : 1 , " streamID " : 1 , " encodeFormat " : "H265" , " resolution " : "3840X2160" , " bitRateControl " : " VBR " , " gop " : 80 , " frameRate " : 20 , " bitRate " : 6000 , " bitRateQuality " : 0 , " qp_enable " : 0 ,​​ " qp_min " :​​ 30 , " qp_max " :​​ 51 }, { " enable " : 1 , " streamID " : 2 , " encodeFormat " : "H265" , " resolution " : "640X360" , " bitRateControl " : " VBR " , " gop " : 80 , " frameRate " : 20 , | , |

37

HTTP API  Instruction Manual

|  | " bitRate " : 700 , " bitRateQuality " : 0 , " qp_enable " : 0 ,​​ " qp_min " :​​ 30 , " qp_max " :​​ 51 }, { " enable " : 0 , " streamID " : 3 , " encodeFormat " : "H265" , " resolution " : "720P" , " bitRateControl " : " VBR " " gop " : 40 , " frameRate " : 10 , " bitRate " : 1000 , " bitRateQuality " : 0 , " qp_enable " : 0 ,​​ " qp_min " :​​ 28 , " qp_max " :​​ 51 } ] } } | , |
| --- | --- | --- |
| Note | The video encoding configuration list starts with json Array returned. Each array member represents the main stream, sub-stream, and triple-stream. The meaning of each element in the array member is: Enable : Enable flag streamID : Bit Stream ID , 1 for Main stream , 2 for Subcode flow , 3 for Three-stream . Note meaning , Depend on At calendar history Original because , this inside of number definition and / HAPI /V1.0/ system / video / capability Interfaces listed stream_type​​ The value definitions are different. encodeFormat : encoding name, value is H264 , H265 , H265+ , MJPEG wait resolution : resolution name, usually in terms of resolution width X High means, for example 2560X1440 , some regular resolutions 1080P/ CIF etc. characters String direct representation bitRateControl : Bit rate control mode, the value is VBR , CBR wait gop : I Frame interval, That is, how many frames are generated every I frame frameRate : frame rate bitRate : bit rate bitRateQuality : video quality , value range 0-5.0​​ Indicates custom, 1 Indicates that the bit rate is set to the lowest allowed bit rate, 2 Indicates that the minimum bit rate is set. The middle value of the default bitrate, 3 Indicates setting the bit rate to the default. 4 Indicates that the bit rate is set to the middle value between the default bit rate and the maximum bit rate. 5 Indicates that the highest code allowed is set Rate qp_enable : Enable customization QP qp_min :​​ customize QP The minimum QP Value , 1-51 qp_max :​ customize QP Maximum QP Value, 1-51 |  |
| Status |  |  |

HTTP API  Instruction Manual

## 2.10.3 Set video encoding configuration / HAPI /V1.0/ system / video / set

| URL | /HAPI/V1.0/system/video/set |
| --- | --- |
| Description | Set video encoding |
| Method | GET/PUT |
| Sample | Set the main stream resolution: http://192.168.1.202/HAPI/V1.0/system/video/set?streamId=1&resolution=3840X2160&username=admin&p​​​​​​​​​​​​​​​​​​​ assword =e10 adc 3949 ba 59 abbe 56e057f20f883e PUT method: PUT / HAPI /V1.0/ system / video / set HTTP /1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache content - type : application / json Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Host: 192.168.1.202 Content-Length: 116 |
|  | { |
|  | "streamId":"1", "resolution":"3840X21 60", "username":"admin", " password ":"e10 adc 3949 ba 59 abbe 56e057f20f883e" |
|  | } |
|  | Set sub-stream encoding, resolution, and frame rate: http://192.168.1.202/HAPI/V1.0/system/video/set?streamId=2&encodeFormat=H264&resolution=VGA&frame​​​​​​​​​​​​​​​​​​​​ eRate = 25& username = admin & password = e10 adc 3949 ba 59 abbe 56e 057f20f883e Set the main stream bit rate and frame rate: Set the playback volume: http :// 192.168.1.202 / HAPI / V1.0 / system / audio / set ? volume _ play = 80 & username = admin & pas sword =e10 adc 3949 ba 59 abbe 56e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | Fill in one or more of the following parameters. An error will be returned if a parameter is missing or the range is incorrect: streamID : Bit Stream ID , 1 for Main stream , 2 for Subcode flow , 3 for Three-stream . Note meaning , Depend on At calendar history Original because , this inside of number definition and / HAPI /V1.0/ system / video / capability Interfaces listed stream_type​​ The value definitions are different. enable : enable flag encodeFormat : encoding name, value is H264 , H265 , H265+ , MJPEG wait resolution : resolution name, usually in terms of resolution width X High means, for example 2560X1440 , some regular resolutions 1080P/ CIF etc. characters String direct representation |

HTTP API  Instruction Manual

|  | bitRateControl : Bit rate control mode, the value is VBR , CBR wait gop : I Frame interval, That is, how many frames are generated every I frame frameRate : frame rate bitRate : bit rate bitRateQuality : video quality , value range 0-5.0​​ Indicates custom, 1 Indicates that the bit rate is set to the lowest allowed bit rate, 2 Indicates that the minimum bit rate is set. The middle value of the default bitrate, 3 Indicates setting the bit rate to the default. 4 Indicates that the bit rate is set to the middle value between the default bit rate and the maximum bit rate. 5 Indicates that the highest code allowed is set Rate qp_enable : Enable customization QP qp_min :​​ customize QP The minimum QP Value , 1-51 qp_max :​ customize QP Maximum QP Value, 1-51 username / password or uid |
| --- | --- |
| Success Response Data | See also 2.10.2GET Returns data. |
| Note | If the audio parameters are set successfully, the returned data will carry the current audio configuration data. |
| Status |  |

## 2.11 Audio Configuration

## 2.11.1   Gain   Pick   sound   frequency   match   Place   Edit   code   able   force   set / HAPI /V1.0/ system / audio / capability

| URL | /HAPI/V1.0/system/audio/capability |  |
| --- | --- | --- |
| Description | Get the supported audio encoding capabilities |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / system / audio / capability ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/system/audio/capability " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : [ { "codec_name" : "G.711" , "channels" : 1 , "bitspersample" : 16 , "samplerate" : 8 , "bitrate" : 64 , | , |

# HTTP API  Instruction Manual

|  | "def_config" : 1 }, { "codec_name" : "AAC" , "channels" : 2 , "bitspersample" : 16 , "samplerate" : 16 , "bitrate" : 16 , "def_config" : 0 } ] } } |
| --- | --- |
| Note | The list of supported video codecs is as follows json Array returned. Meaning of array members : codec_name : encoding name, value is G.711 , G.711A , AAC​ wait channels : number of audio channels bitspersample : the number of sampling bits, that is, the number of bits occupied by each sample samplerate : sampling rate bitrate : bit rate def_config : Whether the encoding type is the default configuration of the camera |
| Status |  |

## 2.11.2 Get audio configuration / HAPI / V1.0 / system / audio / get

| URL | /HAPI/V1.0/system/audio/get |  |
| --- | --- | --- |
| Description | Get audio configuration |  |
| Method | GET/PUT |  |
| Sample | http :// 192.168.1.202 / HAPI / V1.0 / system / audio / get ? username = admin & password = e10 adc 39 49 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0 /system/audio/get" "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : { "enable" : 1 , | , |

# HTTP API  Instruction Manual

|  | "sampleRate" : 8000 , "encodeFormat" : "G.711" "bitRate" : 64000 , "volume_capture" : 90 , "volume_play" : 95 , "amplify" : 1 , "aec_enable" : 1 , "mute_ptz_turn" : 0 } } } | , |
| --- | --- | --- |
| Note | Enable : Enable flag sampleRate : audio encoding sampling rate encodeFormat : encoding name, encoding name, value is G.711 , G.711A , AAC wait bitRate : audio encoding bit rate volume_capture : audio input volume volume_play : audio output volume amplify : Enable the device's internal software amplifier aec_enable : Enable echo cancellation (supported by some devices ) mute_ptz_turn : Enable the mute function when the gimbal turns |  |
| Status |  |  |

## 2.11.3 Set audio configuration / HAPI / V1.0 / system / audio / get

| URL | /HAPI/V1.0/system/audio/get |  |
| --- | --- | --- |
| Description | Get audio configuration |  |
| Method | GET/PUT |  |
| Sample | Set encoding: http :// 192.168.1.202 / HAPI /V1.0/ system / audio / set ? encodeFormat = G.711& username = admin & password = e10 adc 3949b a59 abbe 56e057f20f883e Set the playback volume: http :// 192.168.1.202 / HAPI / V1.0 / system / audio / set ? volume _ play = 80 & username = admin & pas sword =e10 adc 3949 ba 59 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) |  | Fill in one or more of the following parameters. An error will be returned if a parameter is missing or the range is incorrect: enable : Enable audio encoding sampleRate : audio encoding sampling rate encodeFormat : encoding name, encoding name, value is G.711 , G.711A , AAC wait bitRate : audio encoding bit rate volume_capture : audio input volume volume_play : audio output volume amplify : Enable the device's internal software amplifier |

42

# HTTP API  Instruction Manual

|  | aec_enable : Enable echo cancellation (supported by some devices) mute_ptz_turn : Enable the mute function when the PTZ is turning username/password or uid |  |
| --- | --- | --- |
| Success Response Data | See also 2.11.2GET Returns data. |  |
| Note | If the parameters are set successfully, the current configuration is returned |  |
| Status |  |  |

## 2.12 OSD Configuration

## 2.12.1 Acquisition OSD Configuration / HAPI /V1.0/ system / osd / get

| URL | /HAPI/V1.0/system/osd/get​ |
| --- | --- |
| Description | Get video encoding configuration |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/osd/get?username=admin&password=e10adc3949ba59abbe56e057f​​​​​​​​​​​​​​​​​​​​​​​​ 20f883e |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ system / osd / get " , " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " add_overlay "​​ : 3 , " style " : 3 , " DsplayWeek " : 1 , " OverlayFps " : 0 , " fontsize " : 0 , " time 24 or 12" : 0 , " timeOverlay " : { " posType " : 0 , " posX " : 1 , " posY " : 1 , " timeFormat " : " yyyy - mm - dd hh : mm : ss " }, " titleOverlay " : { " posType " : 0 , |

43

# HTTP API  Instruction Manual

|  | " posX " : 0 , " posY " : 0 , " titleType " : 0 , " title_utf8 " :​​ "e6b58 be 8 af 9543616d657261" } } } } |
| --- | --- |
| Note | enable ： OSD Enable add_overlay : Overlay additional information. 0 : None, 1 : Overlay resolution, 2 : Overlay bitrate, 3 : Overlay resolution and bitrate style : Overlay style, the value range refers to the enumeration value definition: typedef enum { AJ_OVERLAY_STYLE_BLACK_WHITE = 0 ,​​​​​​ // Black text on white background AJ_OVERLAY_STYLE_WHITE_BLACK = 1 ,​​​​​​ // White text on black background AJ _ OVERLAY _ STYLE _ TRANSPARENT _ BLACKWHITE = 2, // Transparent background, black text and white frame AJ _ OVERLAY _ STYLE _ TRANSPARENT _ WHITEBLACK = 3, // transparent background, White text black frame AJ _ OVERLAY _ STYLE _ TRANSPARENT _ BLACK = 4, // transparent background, black text AJ _ OVERLAY _ STYLE _ TRANSPARENT _ WHITE = 5, // transparent background, White Characters AJ_OVERLAY_STYLE_INVERSE_COLOR = 6 ,​​​​​​ // Invert color }AjOsdOverlayStyle; DsplayWeek : Displays the day of the week OverlayFps : Overlay frame rate fontsize : overlay text size, 0 : standard 1 : Large font 2 : Extra large font time 24 or 12 ： 0:12 Hour format , 1:24 Hour format timeOverlay : time OSD Configuration titleOverlay : title OSD Configuration posType : coordinate type, 0 express posX 、 posY The configuration is 4 Angular position, 1 express posX 、 posY The configuration is the screen percentage position posX : When the coordinate type is four corner positions, 0 On the left side of the screen, 1 Indicates the right side of the screen, 2 Indicates not to display. When the coordinate type is percentage, the value range is 0-100 , Indicates the position as a percentage of the horizontal size of the picture. posY : When the coordinate type is four-corner position, 0 At the top of the screen, 1 Indicates the bottom of the screen, 2 Indicates not to display. When the coordinate type is percentage, the value range is 0-100 , Indicates the position as a percentage of the vertical size of the screen. timeFormat : time format, value range: " yyyy - mm - dd hh : mm : ss ", " yyyy / mm / dd hh : mm : ss ", " yy - mm - dd hh : mm : ss ", " yy / mm / dd hh : mm : ss ", " hh : mm : ss dd / mm / yyyy ", " hh : mm : ss dd - mm - yyyy ", " hh : mm : ss mm / dd / yyyy ", " hh : mm : ss mm - dd - yyyy ", " mm / dd / yyyy hh : mm : ss ", " mm - dd - yyyy hh : mm : ss " ; |

# HTTP API  Instruction Manual

|  | titleType : title type, 0 Indicates superimposed text, 1 Indicates an overlay image title_utf8 : when​​ titleType for 0 hour, Contents utf8​ Format 16 Hexadecimal format string ( utf- 8 The format cannot be used directly ascii Code string Storage and output, so each byte is 16 Format the binary into 2 Bytes ascii character ) ; when titleType for 1 hour, Content is on camera bmp The absolute path of the image. utf8​ String conversion 16 Binary format string API refer to: int hexdataTohexStr(const char * buf, unsigned int len, char* out, unsigned int outbuflen) { if ( outbuflen == 0 ) return -1; if ( outbuflen <= 2 ) { out [0] = 0; } if ( len >= ( outbuflen -1)/2 ) len = ( outbuflen -1)/2; unsigned int iIndex; for ( iIndex = 0; iIndex < len ; iIndex ++) { sprintf (& out [2* iIndex ], "%02x", ( unsigned char ) buf [ iIndex ]); } return 0; } |
| --- | --- |
| Status |  |

## 2.12.2 Settings OSD Configuration / HAPI /V1.0/ system / osd / set

| URL | /HAPI/V1.0/system/osd/ set |  |
| --- | --- | --- |
| Description | Get video encoding configuration |  |
| Method | PUT |  |
| Sample |  | PUT / HAPI /V1.0/ system / osd / set HTTP /1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache content - type : application / json Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Host: 192.168.1.202 Content-Length: 404 |

HTTP API  Instruction Manual

|  | { } | "username": "admin", "password": "123456", "enable": 1, "add_overlay": 2, "style": 3, "DsplayWeek": 1, "OverlayFps": 0, "fontsize": 0, "time24or12": 0, "timeOverlay": { "posType": 1, "posX": 50, "posY": 50, "timeFormat": "hh:mm:ss mm-dd-yyyy" }, "titleOverlay": { "posType": 0, "posX": 0, "posY": 0, "titleType": 0, "title_utf8": "43616d657261" } |
| --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | Fill in one or more of the following parameters. If a parameter is missing or the range is incorrect, an error will be returned. For parameter value ranges, see 2.12.1 { " username ": " admin ", " password ": "123456", " enable ": 1, " add_overlay " :​ 2, " style ": 3, " DsplayWeek ": 1, " OverlayFps ": 0, " fontsize ": 0, " time 24 or 12 ": 0, " timeOverlay ": { " posType ": 1, " posX ": 50, " posY ": 50, " timeFormat ": " hh : mm : ss mm - dd - yyyy " }, " titleOverlay ": { " posType ": 0, |  |

HTTP API  Instruction Manual

|  | " posX ": 0, " posY ": 0, " titleType ": 0, " title_utf8 " :​ "43616d657261" } } |
| --- | --- |
| Success Response Data | See also 2.12.1GET Returns data. |
| Note | If the parameters are set successfully, the current configuration is returned |
| Status |  |

## 2.13 Intelligent Detection Configuration

## 2.13.1 Get smart detection capability set / HAPI /V1.0/ smart / capability

| URL | /HAPI/V1.0/smart/capability |  |
| --- | --- | --- |
| Description | Get the smart detection capability set |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/capability?username=admin&password=e10adc3949ba59abbe56e057f​​​​​​​​​​​​​​​​​​​​​ 20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / capability " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { "SupportTargetDetect" : 1 , " SupportFaceFd " : 0 , " SupportFaceFr " : 0 , " SupportLPR " : 0 , " SupportVG " : 0 , " SupportRegionAI " : 0 , " SupportAudioDetect " : 1 , " SupportCoverDetect " : 1 } } } | , |
| Note | SupportTargetDetect : Capable of detecting non-targets such as people and vehicles |  |

HTTP API  Instruction Manual

|  | SupportFaceFd : capable of face detection SupportFaceFr : capable of face recognition SupportLPR : Capable of license plate recognition SupportVG : Capable of detecting line mixing SupportRegionAI : Capable of intelligent regional detection (staying, entering, leaving the region, etc.) SupportAudioDetect : Supports abnormal sound detection capability SupportCoverDetect : Support video occlusion detection capability |
| --- | --- |
| Status |  |

## 2.13.2 Obtain Pick Object Detection (People car non-motor vehicle ） Algorithm support hold of Target Class type / HAPI / V1.0 / smart / objectdetect / capability

| URL | /HAPI/V1.0/smart/objectdetect/capability |  |
| --- | --- | --- |
| Description | Get the smart detection capability set |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/objectdetect/capability?username=admin&password=e10adc3949ba5​​​​​​​​​​​​​​​​​​​​​ 9 abbe 56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / objectdetect / capability " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : [ { " type " : 4 } ] } } | , |
| Note | Supported target types are json Array returned, type definition: typedef enum { AI_TYPE_BIT_CAR =​​​​​​ 0, // Car AI_TYPE_BIT_MOTO =​​​​​​ 1, // Motorcycle AI _ TYPE _ BIT _ ELECTRICBICYCLE = 2, // Electric bike AI_TYPE_BIT_BICYCLE =​​​​​​ 3.//Bicycle​ |  |

HTTP API  Instruction Manual

|  | AI_TYPE_BIT_HUMAN = 4 ,​​​​ // Human figure AI_TYPE_BIT_FACE =​​​​​​ 5, // human face AT _ TYPE _ BIT _ NONMOTO _ VEHICLE = 6, // non-motor vehicle AT_TYPE_BIT_FIRE = 7 , // flame​​​ AT _ TYPE _ BIT _ FALLINGOBJECT = 8, // falling objects AI_TYPE_BIT_MAX ,​​​​​​ } AjAiBits ; |
| --- | --- |
| Status |  |

2.13.3   Gain Pick Order Standard Inspection Measurement (people car No machine move car ） Calculate Law branch hold Union move able force

/ HAPI /V1.0/ smart / linkage / capability

| URL | /HAPI/V1.0/smart/linkage/capability |  |
| --- | --- | --- |
| Description | Get the intelligent detection linkage capability set |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/linkage/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e​​​​​​​​​​​​​​​​​​​​​​​​ |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / linkage / capability " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : [ { " SupportLightAction " : 0 }, { " SupportBRAlarmAction " : 0 }, { " SupportAudioAction " : 1 }, { " SupportIOOutAction " : 1 } ] } } | , |

HTTP API  Instruction Manual

| Note | SupportLightAction : Supports flashing of fill light (flashing of fill light when a target is detected) SupportBRAlarmAction : Supports red and blue alarm light linkage SupportAudioAction : Supports alarm sound linkage SupportIOOutAction : Support IO Output linkage |
| --- | --- |
| Status |  |

## 2.13.4 Get the warning tone list / HAPI / V1.0 / smart / audiofiles / get

| URL | /HAPI/V1.0/smart/audiofiles/get |
| --- | --- |
| Description | Get the device warning tone list |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/audiofiles/get?username=admin&password=e10adc3949ba59abbe56e​​​​​​​​​​​​​​​​​​​​​​​ 057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |
| Success Response Data | { " Response ": { " ResponseURL ": "/ HAPI /V1.0/ Smart / audiofiles / get ", " SessionID ": "", " ResponseCode ": 0, " ResponseString ": " Succeed ", " Data ": [ { " filename ": "/ opt / ch / attention _ en . mp 3", "UnicodeDesc": " fffe 5700610072006e002000720065006d0069006e006400650072002c00 200059006f007500200068006100760065002000 65006e007400650072006500640020007400680065002 0006d006f006e00690074006f00720069006e0067002000610072006 5006100" }, { " filename ": "/ opt / ch / attention 03. mp 3", " UnicodeDesc ": " fffec 179 ba 4e8698 df 57f78 bff 526097d18f" }, { " filename ": "/ opt / ch / attention 02. mp 3", " UnicodeDesc ": " fffea 860f25 ddb 8f6551 dd 8f81793a53 df 57f78b3d5 ceb 5 fbb 79005f" }, { " filename ": "/ opt / ch / attention . mp 3", " UnicodeDesc ": " fffe 296 ea 899d0633a79a860f25 ddb 8f6551d17 6a7633a53 df 57" |

HTTP API  Instruction Manual

|  | }, { " filename " : " /opt/ch/Audio.mp3 " }​​​​ ] } } |
| --- | --- |
| Note | Filename The absolute path of the warning sound. UnicodeDesc For the description information of the corresponding file, unicode String 16 Binary format string. |
| Status |  |

## 2.13.5 Get motion detection configuration / HAPI / V1.0 / motiondetect / get

| URL | /HAPI/V1.0/smart/motiondetect/get |  |
| --- | --- | --- |
| Description | Get motion detection configuration |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/motiondetect/get?username=admin&password=e10adc3949ba59abb​​​​​​​​​​​​​​​​​​​​​​ e56e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / motiondetect / get " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, | , |

HTTP API  Instruction Manual

|  | " AreaConfig " : { " block_col " :​​ twenty two , " block_row " :​​ 18 , " blockconfig " : [ { " block_row " :​​ "00011111111111111 11111" }, { " block_row " :​​ "00011111111111111 11111" }, { " block_row " :​​ "00011111111111111 11111" }, { " block_row " :​​ "00011111111111111 11111" }, { " block_row " :​​ "11111111111111111 11111" }, { " block_row " :​​ "11111111111111111 11111" }, { " block_row " :​​ "11111111111111111 11111" }, { " block_row " :​​ "11111111111111111 11111" }, { " block_row " :​​ "111111111111100000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { |
| --- | --- |

HTTP API  Instruction Manual

|  | " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" }, { " block_row " :​​ "00000000000000000 00000" } ] }, " Sensitivity " : 80 , " AlarmThreshold " : 20 , " DayNightSwitch " : 1 , " NightSensitivity " : 80 , " NightAlarmThreshold " : 20 , " NightStartTime " : "00:00:00" , " NightEndTime " : "00:00:00" , " AlarmAction " : { " LightAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " BRAlarmAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmServer " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmPush " : { " ArmingMode " : 4 , " TimeSpanNum " : 1 , " DayPlan " : [ { |
| --- | --- |

HTTP API  Instruction Manual

|  | " startTime " : "00:00:00" , " endTime " : "23:59:59" } ] }, " AudioAction " : { " times " : 1 , " intervalsecnods " : 10 , " filename " : " /opt/ch/attention_en.mp3 "​​​​​​​​ " ArmingSetting " : { " ArmingMode " : 0 , " TimeSpanNum " : 1 , " DayPlan " : [ { " startTime " : "00:00:00" , " endTime " : "23:59:59" } ] } }, " IOOutputAction " : { " channelCnt " : 4 , " channels " : [ { " portIndex " : 1 , " enable " : 0 }, { " portIndex " : 2 , " enable " : 0 }, { " portIndex " : 3 , " enable " : 0 }, { " portIndex " : 4 , " enable " : 0 } ] } } } } | , |
| --- | --- | --- |

HTTP API  Instruction Manual

|  | } |
| --- | --- |
| Note | enable : Enable algorithm detection ArmingFlag : Alarm detection arming configuration, as a general switch to control all linkage outputs, for specific meanings, see 4.1 AreaConfig : Motion detection area configuration, for specific meaning, see 4.3 Sensitivity : Motion detection sensitivity, 0-100 . The larger the value, the easier it is to trigger. AlarmThreshold : threshold, 0-100 , configures the percentage threshold of pixels detected to change in each sub-area. The smaller the value, the easier it is to trigger. DayNightSwitch : Day and night parameter switch, when enabled, the configured night sensitivity and threshold are used at night NightSensitivity : Night motion detection sensitivity NightAlarmThreshold : Night threshold NightStartTime : Night start time NightEndTime : Night end time AlarmAction : Alarm linkage configuration LightAction : Fill light linkage configuration, for specific meanings, see 4.4 BRAlarmAction : Red and blue alarm light linkage configuration: For specific meaning, see 4.4 BRAlarmAction : Red and blue alarm light linkage configuration: For specific meaning, see 4.4 AlarmServer : Linkage reporting server configuration: For details, see 4.4 AlarmPush : Alarm push linkage configuration: For specific meaning, see 4.4 AudioAction : Alarm sound linkage configuration: For details, see 4.5 IOOutputAction : IO Output linkage configuration: For details, see 4.6 |
| Status |  |

## 2.13.6 Set motion detection configuration / HAPI / V1.0 / motiondetect / set

| URL | /HAPI/V1.0/smart/motiondetect/set |
| --- | --- |
| Description | Get motion detection configuration |
| Method | PUT |
| Sample | PUT / HAPI / V1.0 / Smart / motiondetect / set HTTP /1.1 User-Agent: PostmanRuntime-Apip ostRuntime/1.1.0 Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 1175 { "username": "admin", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f883e", "AreaConfig": { "block_col": twenty two, "block_row": 18, |

HTTP API  Instruction Manual

|  | "blockconfig": [ { " block_row " :​ "0001111111111111111111" }, { " block_row " :​ "0001111111111111111111" }, { " block_row " :​ "0001111111111111111111" }, { " block_row " :​ "0001111111111111111111" }, { " block_row " :​ "111111111111111111 1111" }, { " block_row " :​ "111111111111111111 1111" }, { " block_row " :​ "111111111111111111 1111" }, { " block_row " :​ "111111111111111111 1111" }, { " block_row " :​ "1111111111111000 0000000" }, { " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" }, { |
| --- | --- |

HTTP API  Instruction Manual

|  | " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" }, { " block_row " :​ "0000000000000000000000" } ] } } |
| --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.5 Obtained JSON Data, fill in one or more of the following parameters , if the required parameters are missing or the range is incorrect, an error will be returned When the parameter is a separate configuration JSON Structured data ( ArmingFlag / AreaConfi / and AlarmAction Under the node LightAction/BRAlarmAction/AlarmServer/AlarmPush/AudioAction/IOOutputAction , etc.) Fill in all required member parameters. username password enable ArmingFlag AreaConfig Sensitivity AlarmThreshold DayNightSwitch NightSensitivity NightAlarmThreshold NightStartTime NightEndTime AlarmAction |
| Success Response Data | See also 2.13.5GET Returns data. |
| Note | If the parameters are set successfully, the current configuration is returned. |
| Status |  |

## 2.13.7 Get object detection configuration / HAPI / V1.0 / smart / objectdetect / get

| URL | /HAPI/V1.0/smart/objectdetect/get |
| --- | --- |
| Description | Get target detection configuration |
| Method | GET/PUT |

HTTP API  Instruction Manual

| Sample | http://192.168.1.202/HAPI/V1.0/Smart/objectdetect/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e​​​​​​​​​​​​​​​​​​​​​​​ |  |
| --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / objectdetect / get " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , "type" : { "CAR" : 0 , " MOTO " : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 1 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, " Polygon " : { " PointCnt " : 7 , " Points " : [ { "x" : 17 , "y" : 11 }, { | , |

HTTP API  Instruction Manual

|  | "x" : 86 , "y" : 13 }, { "x" : 95 , "y" : 83 }, { "x" : 62 , "y" : 94 }, { "x" : 16 , "y" : 90 }, { "x" : 5 , "y" : 37 }, { "x" : 8 , "y" : 14 } ] }, " Sensitivity " : 5 , " AlarmThreshold " : 75 , " minTargetRate " : 0 , " nonMotionFilter " : 1 , " AlarmAction " : { " LightAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " BRAlarmAction " : { " ArmingMode " : 1 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmServer " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] |
| --- | --- |

HTTP API  Instruction Manual

|  | }, " AlarmPush " : { " ArmingMode " : 1 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AudioAction " : { " times " : 1 , " intervalsecnods " : 10 , " filename " : " /opt/ch/attention.mp3 " ,​​​​​​ " ArmingSetting " : { " ArmingMode " : 4 , " TimeSpanNum " : 3 , " DayPlan " : [ { " startTime " : "01:00:00" , " endTime " : "13:59:00" }, { " startTime " : "15:00:00" , " endTime " : "17:00:00" }, { " startTime " : "21:00:00" , " endTime " : "22:00:00" } ] } }, " IOOutputAction " : { " channelCnt " : 4 , " channels " : [ { " portIndex " : 1 , " enable " : 0 }, { " portIndex " : 2 , " enable " : 0 }, { " portIndex " : 3 , " enable " : 0 }, |
| --- | --- |

HTTP API  Instruction Manual

|  | { " portIndex " : 4 , " enable " : 0 } ] }, " draw _ rect _ enable " : 1 , " draw _ human _ enable " : 1 , " track_human_enable "​​​​ : 0 , " rect_twinkle_enable "​​​​ : 0 , " auto_zoom_enable "​​​​ : 0 , " gunball_track_mode "​​​​ : 0 } } } } |
| --- | --- |
| Note | " enable ": algorithm enable, control algorithm detection, used to implement the camera's soft light-sensitive switch light and other logic, And notify other linkage modules of the alarm " type ": Target detection type, see 4.7 " ArmingFlag ": Algorithm arming master switch, configurable 7x24 weekly plan " Polygon ": detection area " Sensitivity ": sensitivity, value range 0-10 " AlarmThreshold ": alarm threshold (not used yet) " minTargetRate ": The minimum target detection screen ratio, 0-100 " nonMotionFilter ": no motion, no check, 0-1 " LightAction ": Fill light linkage, flashing fill light when alarm occurs " BRAlarmAction ": The red and blue warning lights are linked, and the red and blue warning lights of the dedicated interface flash when there is an alarm. " AlarmServer ": Report to the alarm center " AlarmPush ": Push to APP " AudioAction ": Linked alarm sound " IOOutputAction ": linkage IO Output " draw _ rect _ enable ": Display the detection area " draw _ human _ enable ": Display detection targets " track_human_enable " : Enable human tracking​ " rect_twinkle_enable " : flickering detection box​ " auto_zoom_enable " :​​​ Automatic zoom tracking " gunball_track_mode " : Enable gunball tracking​ |
| Status |  |

## 2.13.8 Set object detection configuration / HAPI / V1.0 / smart / objectdetect / set

| URL | /HAPI/V1.0/smart/objectdetect/set |
| --- | --- |
| Description | Setting up object detection configuration |

HTTP API  Instruction Manual

| Method | PUT |
| --- | --- |
| Sample | PUT /HAPI/V1.0/Smart/objectdetect/set HTTP/1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 294 { "username": "admin", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f883e", "ArmingFlag": { "ArmingMode": 2, "WeekPlan": { "day0": 16777211, "day1": 16777212, "day2": 16777213, "day3": 16777214, "day4": 16777215, "day5": 16777216, "day6": 16777217 } } } |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.7 Obtained JSON Data, fill in one or more of the following parameters , if the required parameters are missing or the range is incorrect, an error will be returned When the parameter is a separate configuration JSON Structured data ( ArmingFlag/ Polygon / and AlarmAction Under the node LightAction/BRAlarmAction/AlarmServer/AlarmPush /AudioAction/IOOutputAction etc.), you need to complete Fill in all required member parameters. username password enable type ArmingFlag Polygon Sensitivity AlarmThreshold minTargetRate nonMotionFilter AlarmAction |
| Success Response Data | See also 2.13.7 GET Returns data. |
| Note | If the parameters are set successfully, the current configuration is returned. |

HTTP API  Instruction Manual

| Status |  |
| --- | --- |

## 2.13.9 Get video cover configuration / HAPI / V1.0 / smart / videocover / get

| URL | /HAPI/V1.0/smart/videocover/get |  |
| --- | --- | --- |
| Description | Get video mask configuration |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/videocover/get?username=admin&password=e10adc3949ba59abbe56​​​​​​​​​​​​​​​​​​​​​​​​ e057f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data |  | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / videocover / get " , " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " Sensitivity " : 20 , " Threadhold_second " : 5 ,​​ " BackgroundUpdateSecond " : 40 , " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, " AlarmAction " : { " AudioAction " : { " times " : 1 , " intervalsecnods " : 10 , , " filename " : " /opt/ch/attention_en.mp3 "​​​​​​​​ " ArmingSetting " : { " ArmingMode " : 1 , " TimeSpanNum " : 1 , |

HTTP API  Instruction Manual

|  | "DayPlan" : [ { " startTime " : "00:00:00" " endTime " : "23:59:59" } ] } }, " IOOutputAction " : { " channelCnt " : 4 , " channels " : [ { " portIndex " : 1 , " enable " : 1 }, { " portIndex " : 2 , " enable " : 0 }, { " portIndex " : 3 , " enable " : 0 }, { " portIndex " : 4 , " enable " : 0 } ] } } } } } | , |
| --- | --- | --- |
| Note | If the parameters are set successfully, the current configuration is returned |  |
| Status |  |  |

## 2.13.10 Set video cover configuration / HAPI /V1.0/ smart / videocover / set

| URL | /HAPI/V1.0/smart/videocover/set |
| --- | --- |
| Description | Set video mask configuration |
| Method | PUT |
| Sample | PUT / HAPI / V1.0 / Smart / videocover / set HTTP /1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 |

HTTP API  Instruction Manual

|  | Cache-Control: no-cache Accept : */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host : 192.168.1.202 Content - Length : 91 { " username ": " admin ", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f883e", " enable ": 1 } |
| --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.9 Obtained JSON Data, fill in one or more of the following parameters , if the required parameters are missing or the range is incorrect, an error will be returned When the parameter is a separate configuration JSON When structuring data, all required member parameters need to be fully filled. username password enable Sensitivity Threadhold_second BackgroundUpdateSecond ArmingFlag Sensitivity AlarmAction |
| Success Response Data | See also 2.13.9 GET Returned data |
| Note | If the parameters are set successfully, the current configuration is returned |
| Status |  |

## 2.13.11 Get face detection configuration / HAPI /V1.0/ smart / facedetect / get

| URL | /HAPI/V1.0/smart/facedetect/get |  |  |
| --- | --- | --- | --- |
| Description | Get face detection configuration |  |  |
| Method | GET/PUT |  |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/facedetect/get?username=admin&password=e10adc3949ba59abbe56​​​​​​​​​​​​​​​​​​​​​​​​ e057f20f883e |  |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |  |
| Success Response Data | { " Response " : { "ResponseURL" : " SessionID " : "" "ResponseCode" : | "/ HAPI /V1.0/ Smart / facedetect / get " , 0 , | , |

HTTP API  Instruction Manual

|  | " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, "Area" : { "x" : 0 , "y" : 0 , "w" : 50 , "h" : 90 }, " Sensitivity " : 0 , " AlarmThreshold " : 75 , " AlarmAction " : { " draw _ rect _ enable " : 1 , " AudioAction " : { " times " : 1 , " intervalsecnods " : 0 , " filename " : " /opt/ch/attention.mp3 " ,​​​​​​ " ArmingSetting " : { " ArmingMode " : 1 , " TimeSpanNum " : 1 , " DayPlan " : [ { " startTime " : "00:00:00" , " endTime " : "23:59:59" } ] } }, " IOOutputAction " : { " channelCnt " : 4 , " channels " : [ { |
| --- | --- |

HTTP API  Instruction Manual

|  | " portIndex " : 1 , " enable " : 0 }, { " portIndex " : 2 , " enable " : 0 }, { " portIndex " : 3 , " enable " : 0 }, { " portIndex " : 4 , " enable " : 0 } ] } } } } } |
| --- | --- |
| Note | " enable ": algorithm enabled " ArmingFlag ": algorithm arming master switch, configurable 7x24 weekly plan " Area ": Detect rectangular area " Sensitivity ": sensitivity, value range 0-100 " AlarmThreshold ": detection threshold " AudioAction ": Linked alarm sound " IOOutputAction ": linkage IO Output " draw _ rect _ enable ": Display the detection area |
| Status |  |

## 2.13.12 Set face detection configuration / HAPI / V1.0 / smart / facedetect / set

| URL | /HAPI/V1.0/smart/facedetect/set |
| --- | --- |
| Description | Set up face detection configuration |
| Method | PUT |
| Sample | PUT / HAPI / V1.0 / Smart / facedetect / set HTTP /1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept : */* Accept-Encoding: gzip, deflate, br Connection: keep-alive |

67

HTTP API  Instruction Manual

|  | Content-Type: application/x-www-form-urlencoded Host : 192.168.1.202 Content - Length : 1043 { " username ": " admin ", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f883e", " enable ": 1, " ArmingFlag ": { " ArmingMode ": 1, " WeekPlan ": { " day 0 ": 16777215, " day 1": 16777215, " day 2": 16777215, " day 3": 16777215, " day 4": 16777215, " day 5 ": 16777215, " day 6": 16777215 } }, " Area ": { "x": 0, "y": 0, "w": 50, "h": 90 }, " Sensitivity ": 0, " AlarmThreshold ": 75, " AlarmAction ": { " draw _ rect _ enable ": 1, " AudioAction ": { " times ": 1, " intervalsecnods ": 0, " filename ": "/ opt / ch / attention . mp 3", " ArmingSetting ": { " ArmingMode ": 1, " TimeSpanNum ": 1, " DayPlan ": [ { " startTime ": "00:00:00", " endTime ": "23:59:59" } ] } |
| --- | --- |

HTTP API  Instruction Manual

|  | }, " IOOutputAction ": { " channelCnt ": 4, " channels ": [ { " portIndex ": 1, " enable ": 0 }, { " portIndex ": 2, " enable ": 0 }, { " portIndex ": 3, " enable ": 0 }, { " portIndex ": 4, " enable ": 0 } ] } } } |
| --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.11 Obtained JSON Data, fill in one or more parameters in the parameters, missing required parameters or incorrect range will return an error. When the parameter is a separate configuration JSON When structuring data , all required member parameters need to be fully filled. |
| Success Response Data | refer to 2.13.11 Obtained JSON data |
| Note | If the parameters are set successfully, the current configuration is returned |
| Status |  |

## 2.13.13 Get out-of-bounds detection configuration / HAPI / V1.0 / smart / videogate / get

| URL | /HAPI/V1.0/smart/videogate/get |
| --- | --- |
| Description | Get the out-of-bounds detection configuration |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/videogate/get?username=admin&password=e10adc3949ba59abbe56​​​​​​​​​​​​​​​​​​​​​​​​ e057f20f883e |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |
| Success Response Data | { " Response " : { |

HTTP API  Instruction Manual

|  | " ResponseURL " : "/ HAPI /V1.0/ Smart / videogate / get " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, " Rules " : [ { " enable " : 1 , " Sensitivity " : 80 , "type" : { "CAR" : 1 , "MOTO" : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 0 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " direction " : 2 , "x0" : 0 , "y0" : 0 , "x1" : 0 , "y1" : 100 }, { " enable " : 1 , " Sensitivity " : 0 , "type" : { "CAR" : 0 , | , |
| --- | --- | --- |

HTTP API  Instruction Manual

|  | " MOTO " : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 0 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " direction " : 0 , "x0" : 0 , "y0" : 0 , "x1" : 0 , "y1" : 0 }, { " enable " : 0 , " Sensitivity " : 0 , "type" : { "CAR" : 0 , "MOTO" : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 0 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " direction " : 0 , "x0" : 0 , "y0" : 0 , "x1" : 0 , "y1" : 0 }, { " enable " : 0 , " Sensitivity " : 0 , "type" : { "CAR" : 0 , "MOTO" : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 0 , |
| --- | --- |

HTTP API  Instruction Manual

|  | " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " direction " : 0 , "x0" : 0 , "y0" : 0 , "x1" : 0 , "y1" : 0 } ], " AlarmAction " : { " LightAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " BRAlarmAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmServer " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmPush " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AudioAction " : { " times " : 0 , " intervalsecnods " : 0 , "" " filename " : , " ArmingSetting " : { " ArmingMode " : 0 , " TimeSpanNum " : 1 , " DayPlan " : [ { " startTime " : "00:00:00" " endTime " : "23:59:59" | , |
| --- | --- | --- |

HTTP API  Instruction Manual

|  | } ] } }, " IOOutputAction " : { " channelCnt " : 4 , " channels " : [ { " portIndex " : 1 , " enable " : 0 }, { " portIndex " : 2 , " enable " : 0 }, { " portIndex " : 3 , " enable " : 0 }, { " portIndex " : 4 , " enable " : 0 } ] }, " draw _ rect _ enable " : 0 , " draw _ target _ enable " : 0 } } } } |
| --- | --- |
| Note | " enable ": algorithm enabled " ArmingFlag ": Algorithm arming master switch, configurable 7x24 weekly plan " Rules ": Configure detection rules. You can configure up to 4 , see 4.10 " type ": Target detection type, see 4.7 " LightAction ": Fill light linkage, flashing fill light when alarm occurs " BRAlarmAction ": The red and blue warning lights are linked, and the red and blue warning lights of the dedicated interface flash when there is an alarm. " AlarmServer ": Report to the alarm center " AlarmPush ": Push to APP " AudioAction ": Linked alarm sound " IOOutputAction ": linkage IO Output " draw _ rect _ enable ": Display the detection area " draw _ target _ enable ": Display the detection target |

HTTP API  Instruction Manual

| Status |  |
| --- | --- |

## 2.13.14 Set the out-of-bounds detection configuration / HAPI / V1.0 / smart / videogate / set

| URL | /HAPI/V1.0/smart/videogate/set |  |
| --- | --- | --- |
| Description | Set the out-of-bounds detection configuration |  |
| Method | PUT |  |
| Sample | PUT / HAPI / V1.0 / Smart / videogate / set HTTP /1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept : */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host : 192.168.1.202 Content - Length : 476 { " username " : " admin " , " password " : "e10 adc 3949 ba 59 abbe 56e057f20f883e" , " enable " : 1 , " ArmingFlag " : { " ArmingMode " : 1 }, " Rules " : [ { " enable " : 1 , " Sensitivity " : 80 , "type" : { "CAR" : 1 , " HUMAN " : 1 }, " direction " : 2 , "x0" : 0 , "y0" : 0 , "x1" : 100 , "y1" : 100 } ] } |  |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.13 Obtained JSON Data, fill in one or more parameters in the parameters, missing required parameters or incorrect range will return an error. When the parameter is a separate configuration JSON When structuring data , all required member parameters need to be fully filled. |  |
| Success Response Data |  | refer to 2.13.13 Obtained JSON data |

HTTP API  Instruction Manual

| Note | If the parameters are set successfully, the current configuration is returned |
| --- | --- |
| Status |  |

## 2.13.15 Get region ( perimeter ) detection configuration / HAPI / V1.0 / smart / regionai / get

| URL | /HAPI/V1.0/smart/videogate/get |  |
| --- | --- | --- |
| Description | Get area ( perimeter ) detection configuration |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/regionai/get?username=admin&password=e10adc3949ba59abbe56e0​​​​​​​​​​​​​​​​​​​​​​​​ 57f20f883e |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / regionai / get " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, " Polygon " : { " PointCnt " : 4 , " Points " : [ { "x" : 0 , "y" : 0 }, { "x" : 100 , "y" : 0 }, | , |

HTTP API  Instruction Manual

|  | { "x" : 100 , "y" : 100 }, { "x" : 0 , "y" : 100 } ] }, " Rules " : [ { " enable " : 1 , " Sensitivity " : 80 , "type" : { "CAR" : 1 , "MOTO" : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 1 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " mode " : " LEAVE " , " stayseconds " : 5 }, { " enable " : 0 , " Sensitivity " : 0 , "type" : { "CAR" : 0 , "MOTO" : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 0 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " mode " : " STAY " , " stayseconds " : 0 |
| --- | --- |

HTTP API  Instruction Manual

|  | }, { " enable " : 0 , " Sensitivity " : 0 , "type" : { "CAR" : 0 , "MOTO" : 0 , " ELECTRICBICYCLE " : 0 , " BICYCLE " : 0 , " HUMAN " : 0 , " FACE " : 0 , " NONMOTO_VEHICLE " : 0 ,​​ " FIRE " : 0 , " FALLINGOBJECT " : 0 }, " mode " : " STAY " , " stayseconds " : 0 } ], " AlarmAction " : { " LightAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " BRAlarmAction " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmServer " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmPush " : { " ArmingMode " : 0 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AudioAction " : { " times " : 0 , " intervalsecnods " : 0 , "" " filename " : , |
| --- | --- |

HTTP API  Instruction Manual

|  | " ArmingSetting " : { " ArmingMode " : 0 , " TimeSpanNum " : 1 , " DayPlan " : [ { " startTime " : "00:00:00" " endTime " : "23:59:59" } ] } }, " IOOutputAction " : { " channelCnt " : 4 , " channels " : [ { " portIndex " : 1 , " enable " : 0 }, { " portIndex " : 2 , " enable " : 0 }, { " portIndex " : 3 , " enable " : 0 }, { " portIndex " : 4 , " enable " : 0 } ] }, " draw _ rect _ enable " : 0 , " draw _ target _ enable " : 0 } } } } | , |
| --- | --- | --- |
| Note | " enable ": algorithm enabled " ArmingFlag ": Algorithm arming master switch, configurable 7x24 weekly plan " Polygon ": detection area " Rules ": Configure detection rules. You can configure up to 3 , see 4.11 " type ": Target detection type, see 4.7 " LightAction ": Fill light linkage, flashing fill light when alarm occurs |  |

HTTP API  Instruction Manual

|  | " BRAlarmAction ": The red and blue warning lights are linked, and the red and blue warning lights of the dedicated interface flash when there is an alarm. " AlarmServer ": Report to the alarm center " AlarmPush ": Push to APP " AudioAction ": Linked alarm sound " IOOutputAction ": linkage IO Output " draw _ rect _ enable ": Display the detection area " draw _ target _ enable ": Display the detection target |
| --- | --- |
| Status |  |

## 2.13.16 Set region ( perimeter ) detection configuration / HAPI / V1.0 / smart / regionai / set

| URL | /HAPI/V1.0/smart/regionai/set |
| --- | --- |
| Description | Set area ( perimeter ) detection configuration |
| Method | PUT |
| Sample | PUT / HAPI / V1.0 / Smart / regionai / set HTTP /1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept : */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host : 192.168.1.202 Content - Length : 659 { " username ": " admin ", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f883e", " enable ": 1, " ArmingFlag ": { " ArmingMode ": 1 }, " Polygon ": { " PointCnt ": 4, " Points ": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { |

HTTP API  Instruction Manual

|  | "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, " Rules ": [ { " enable ": 1, " Sensitivity ": 80, " type ": { " CAR ": 1, " MOTO ": 0, " ELECTRICBICYCLE ": 0, " BICYCLE ": 0, " HUMAN ": 1, " FACE ": 0, " NONMOTO_VEHICLE " : 0, " FIRE ": 0, " FALLINGOBJECT ": 0 }, " mode ": " LEAVE ", " stayseconds ": 5 } ] } |  |
| --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.15 Obtained JSON Data, fill in one or more parameters in the parameters, missing required parameters or incorrect range will return an error. When the parameter is a separate configuration JSON When structuring data , all required member parameters need to be fully filled. |  |
| Success Response Data | refer to 2.13.15 Obtained JSON data |  |
| Note | If the parameters are set successfully, the current configuration is returned |  |
| Status |  |  |

## 2.13.17 Get license plate recognition configuration / HAPI /V1.0/ smart / lpr / get

| URL | /HAPI/V1.0/smart/lpr/get |
| --- | --- |
| Description | Get license plate recognition configuration |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/lpr/get?username=admin&password=e10adc3949ba59abbe56e057f20​​​​​​​​​​​​​​​​​​​​​​​ f883e |

HTTP API  Instruction Manual

| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| --- | --- | --- |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Smart / lpr / get " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { " enable " : 1 , " Sensitivity " : 80 , " detectionmode " : 0 , " actionInterval " : 5 , " snapQuality " : " Default " , " ArmingFlag " : { " ArmingMode " : 1 , " WeekPlan " : { " day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215 } }, " Polygon " : { " PointCnt " : 4 , " Points " : [ { "x" : 0 , "y" : 0 }, { "x" : 100 , "y" : 0 }, { "x" : 100 , "y" : 100 }, { "x" : 0 , | , |

HTTP API  Instruction Manual

|  | "y" : 100 } ] }, " AlarmAction " : { " AlarmServer " : { " ArmingMode " : 1 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " AlarmPush " : { " ArmingMode " : 1 , " TimeSpanNum " : 0 , " DayPlan " : [] }, " draw _ rect _ enable " : 1 , " draw _ target _ enable " : 1 , " draw _ osd _ enable " : 1 , " play_voice_enable " : 1​​​​ } } } } |
| --- | --- |
| Note | " enable ": algorithm enable, control algorithm detection, used to implement the camera's soft light-sensitive switch light and other logic, And notify other linkage modules of the alarm " ArmingFlag ": Algorithm arming master switch, configurable 7x24 weekly plan " Polygon ": detection area " Sensitivity ": sensitivity, value range 0-100 " detectionmode ": push image mode, 0: optimal push image 1: Push the map at intervals " actionInterval ": The time interval between two reports for the same license plate , in seconds " snapQuality ": Capture quality ( resolution ) , Default , 1280X720, other resolutions will be supported as needed " draw _ rect _ enable ": draw the detection area " draw _ target _ enable ": draw the detection target " draw _ osd _ enable ": draw the recognized license plate " play_voice_enable " : Voice broadcast of license plate |
| Status |  |

## 2.13.18 Set license plate recognition configuration / HAPI /V1.0/ smart / lpr / set

| URL | /HAPI/V1.0/smart/lpr/set |
| --- | --- |
| Description | Set up license plate recognition configuration |
| Method | PUT |
| Sample | PUT / HAPI /V1.0/ Smart / lpr / set HTTP /1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 |

82

HTTP API  Instruction Manual

|  | Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 663 { "username": "admin", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f88 3e", "enable": 1, "Sensitivity": 80, "detectionmode": 0, "actionInterval": 5, "snapQuality": "Default", "ArmingFlag": { "ArmingMode": 1 }, "Polygon": { "PointCnt": 4, "Points": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, "AlarmAction": { "AlarmServer": { "ArmingMode": 1 }, |
| --- | --- |

HTTP API  Instruction Manual

|  | } | "AlarmPush": { "ArmingMode": 1 }, "draw_rect_enable": 1, "draw_target_enable": 1, "draw_osd_enable": 1, "play_voice_enable": 1 } |
| --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.17 Obtained JSON Data, fill in one or more parameters in the parameters, missing required parameters or incorrect range will return an error. When the parameter is a separate configuration JSON When structuring data , all required member parameters need to be fully filled. |  |
| Success Response Data | refer to 2.13.17 Obtained JSON data |  |
| Note | If the parameters are set successfully, the current configuration is returned. |  |
| Status |  |  |

## 2.13.19 Get fire detection configuration / HAPI / V1. 0 / smart / flameflumes / get

| URL | /HAPI/V1.0/smart/flameflumes/get |  |
| --- | --- | --- |
| Description | Get Fireworks Configuration |  |
| Method | GET/PUT |  |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/flameflumes/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e​​​​​​​​​​​​​​​​​​​​​​​ |  |
| URL params ( simple parameters ) Input Data(PUT) | username/password or uid |  |
| Success Response Data | { "Response" : { "ResponseURL" : "/HAPI/V1.0/Smart/flameflumes/get " "SessionID" : "" , "ResponseCode" : 0 , "ResponseString" : "Succeed" , "Data" : { "enable" : 1 , "Sensitivity" : 60 , "actionInterval" : 5 , "snapQuality" : "1280X720" , "ArmingFlag" : { "ArmingMode" : 0 , "WeekPlan" : { "day0" : 16777215 , "day1" : 16777215 , "day2" : 16777215 , | , |

HTTP API  Instruction Manual

|  | "day3" : 16777215 , "day4" : 16777215 , "day5" : 16777215 , "day6" : 16777215 } }, "Polygon" : { "PointCnt" : 4 , "Points" : [ { "x" : 0 , "y" : 0 }, { "x" : 100 , "y" : 0 }, { "x" : 100 , "y" : 100 }, { "x" : 0 , "y" : 100 } ] }, "AlarmAction" : { "LightAction" : { "ArmingMode" : 0 , "TimeSpanNum" : 0 , "DayPlan" : [] }, "BRAlarmAction" : { "ArmingMode" : 0 , "TimeSpanNum" : 0 , "DayPlan" : [] }, "AlarmServer" : { "ArmingMode" : 1 , "TimeSpanNum" : 0 , "DayPlan" : [] }, "AlarmPush" : { |
| --- | --- |

HTTP API  Instruction Manual

|  | "ArmingMode" : 0 , "TimeSpanNum" : 0 , "DayPlan" : [] }, "AudioAction" : { "times" : 1 , "intervalsecnods" : 0 , "filename" : "/opt/ch/Welcome.mp3" , "ArmingSetting" : { "ArmingMode" : 0 , "TimeSpanNum" : 1 , "DayPlan" : [ { "startTime" : "00:00:00" "endTime" : "23:59:59" } ] } }, "IOOutputAction" : { "channelCnt" : 4 , "channels" : [ { "portIndex" : 1 , "enable" : 0 }, { "portIndex" : 2 , "enable" : 0 }, { "portIndex" : 3 , "enable" : 0 }, { "portIndex" : 4 , "enable" : 0 } ] }, "draw_rect_enable" : 1 , "draw_target_enable" : 1 } } | , |
| --- | --- | --- |

HTTP API  Instruction Manual

|  | } } |
| --- | --- |
| Note | " enable ": algorithm enable, control algorithm detection, used to implement the camera's soft light-sensitive switch light and other logic, And notify other linkage modules of the alarm " ArmingFlag ": Algorithm arming master switch, configurable 7x24 weekly plan " Polygon ": detection area " Sensitivity ": sensitivity, value range 0-100 " actionInterval ": The time interval between two reports for the same license plate , in seconds " snapQuality ": Capture quality ( resolution ) , Default , 1280X720, other resolutions will be supported as needed " draw _ rect _ enable ": draw the detection area " draw _ target _ enable ": draw the detection target " LightAction ": Fill light linkage, flashing fill light when alarm occurs " BRAlarmAction ": The red and blue warning lights are linked, and the red and blue warning lights of the dedicated interface flash when there is an alarm. " AlarmServer ": Report to the alarm center " AlarmPush ": Push to APP " AudioAction ": Linked alarm sound " IOOutputAction ": linkage IO Output |
| Status |  |

## 2.13.20 Set the fire detection configuration / HAPI / V1.0 / smart / flameflumes / set

| URL | /HAPI/V1.0/smart/flameflumes/set |
| --- | --- |
| Description | Setting up the fire detection configuration |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/flameflumes/set HTTP/1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 452 { "username": "admin", " password ": "e10 adc 3949 ba 59 abbe 56e057f20f883e", "enable": 1, "Sensitivity": 60, "actionInterval": 5, "snapQuality": "1280X720", "ArmingFlag": { "ArmingMode": 0 |

87

HTTP API  Instruction Manual

|  | } | }, "AlarmAction": { "AlarmServer": { "ArmingMode": 1 }, "AlarmPush": { "ArmingMode": 0 }, "AudioAction": { "ArmingSetting": { "ArmingMode": 0 } }, "draw_rect_enable": 1, "draw_target_enable": 1 } |
| --- | --- | --- |
| URL params ( simple parameters ) Input Data(PUT) | refer to 2.13.19 Obtained JSON Data, fill in one or more parameters in the parameters, missing required parameters or incorrect range will return an error. When the parameter is a separate configuration JSON When structuring data , all required member parameters need to be fully filled. |  |
| Success Response Data | refer to 2.13.19 Obtained JSON data |  |
| Note | If the parameters are set successfully, the current configuration is returned. |  |
| Status |  |  |

HTTP API  Instruction Manual

# 3 event

服务端

| 订阅注册 /HAPI/V1.0/Event/subscription/regist 服务端返回成功 |  |
| --- | --- |
| 事件通知 |  |
| /HAPI/V1.0/Event/Notification |  |
| 订阅刷新 /HAPI/V1.0/Event/subscription/refresh 服务端返回成功 |  |
| 事件通知 |  |
| /HAPI/V1.0/Event/Notification |  |
| 订阅删除 /HAPI/V1.0/Event/subscription/delete 服务端返回成功 |  |

3.1 Event Workflow

| Client |  |
| --- | --- |

Flow Description Alarm reporting

step 1 : Client is turned on TCP service is used to receive server-side event push

step Steps 2 : guest household end Towards set up Preparation side (Clothes Service end ） order read tell police , catch mouth detailed Affection Ginseng See Note book order read /HAPI/V1.0/Event/subscription/regist

step 3 : The parameters returned by the device side (server) represent the subscription identifier ( ID ), subscription start time ( CurrentTime ) and Subscription information such as the termination time ( TerminationTime ) and the subscription identifier ( ID ) are used to refresh or cancel subsequent alarms.

step 4 : The client needs to send a refresh interface regularly before the subscription end time (TerminationTime) to refresh the subscription . step 5 : The device side (server) returns the refreshed subscription time.

step 6 : If an alarm occurs on the device side (server side), the device side TCP The short connection pushes events to the address specified by the client. See event notification for details. Event notification follows HTTP Agreement, through JSON Carry event information.

Alarm Cancel

step 1 : The client cancels the alarm. For interface details, see Delete Subscription /HAPI/V1.0/ Event/subscription/delete  step 2 : The server cancels successfully and no more alerts are pushed.

HTTP API  Instruction Manual

## 3.2 Event Subscription

## 3.2.1Registration subscription / HAPI /V1.0/ Event / subscription / regist

| URL | / HAPI / V1.0 / Event / subscription / regist |  |  |
| --- | --- | --- | --- |
| Description | Sign up for a subscription |  |  |
| Method | POST |  |  |
| Sample | POST / HAPI /V1.0/ Event / subscription / regist ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e HTTP/1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 107 |  |  |
|  | { |  |  |
|  | "ServerType":0, "ServerName": " 192.168.1.253 ", "Port": 9998, "Duration": 3600, "EventType": "all" |  |  |
|  | } |  |  |
| URL params ( simple parameters ) Input Data(POST) | { " ServerType " : 0 , " ServerName " : " 192.168.1.253 " " Port " : 9998 , " Duration " : 3600 , " PostURLPrefix " : "" , " EventType " : " all " } | , |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Event / subscription / regist " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { "ID" : 2 , |  | , |

HTTP API  Instruction Manual

|  | " ServerType " : 9998 , " ServerName " : "192.168.1.253" , " ServerPort " : 9998 , " EventType " : " all " , " CurrentTime " : 1699930618 , " TerminationTime " : 1699934218 } } } |
| --- | --- |
| Note | The registration subscription request on the device to the same server and port will not create a new duplicate subscription. |
| Status |  |

| Param | Request | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ServerType | M | Enum | IP Address Type: 0: IPv4 ; 1: Domain name | 0 |
| ServerName | M | string | IPv4 Address or domain name. The length range is [0,64] . |  |
| Port | M | unsigned long | Port, in the range [1, 65535] . |  |
| Duration | M | unsigned long | Subscription period, in s , the range is [30, 36 00] . |  |
| PostURLPrefix | C | string | Equipment reporting event Need to add URL forward Suffix, used to specify the path of the alarm event server. | 0 |
| EventType | C | string | Subscription event type, if not filled or filled with "all" means All events need to be reported, and different events are separated by commas. segmentation. | "Motion Detect,O bjectDet ect” |
| ID | M | unsigned long | The subscription ID returned by the device is used to identify which subscription Read, refresh subscription, delete subscription operations need to carry this ID , to indicate which subscription needs to be operated. | 0 |
| CurrentTime | M | unsigned long | UTC Time, from 1970 Year 1 moon 1st​ 0:00​ The number of seconds from the start. | 1477104 900 |
| TerminationTime | M | unsigned long | UTC Time, from 1970 Year 1 moon 1st​ 0:00​ The number of seconds from the start. | 1477104 949 |

## 3.2.2 Refresh subscription / HAPI / V1.0 / Event / subscription / refresh

| URL | / HAPI /V1.0/ Event / subscription / refresh |
| --- | --- |
| Description | Refresh Subscription |

HTTP API  Instruction Manual

| Method | POST |  |
| --- | --- | --- |
| Sample | POST / HAPI /V1.0/ Event / subscription / refresh ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e HTTP/1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 40 { "ID": 1, "Duration": 3600 } |  |
| URL params ( simple parameters ) Input Data(POST) | { " ID " : 2 , " Duration " : 3600 } |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Event / subscription / refresh " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { "ID" : 2 , " ServerType " : 9998 , " ServerName " : "192.168.1.253" , " ServerPort " : 9998 , " EventType " : " all " , " CurrentTime " : 1699931648 , " TerminationTime " : 1699935248 } } } | , |
| Note | Parameter definition see 3.2.1 |  |
| Status |  |  |

HTTP API  Instruction Manual

## 3.2.3 Delete subscription / HAPI / V1.0 / Event / subscription / delete

| URL | / HAPI / V1.0 / Event / subscription / delete |  |
| --- | --- | --- |
| Description | Deleting a subscription |  |
| Method | POST |  |
| Sample | POST / HAPI /V1.0/ Event / subscription / delete ? username = admin & password = e10 adc 3949 ba 59 abbe 56e057f20f883e HTTP/1.1 User-Agent: PostmanRuntime -ApipostRuntime/1.1.0 Cache - Control : no - cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content - Type : application /x- www - form - urlencoded Host: 192.168.1.202 Content-Length: 17 { "ID": 2 } |  |
| URL params ( simple parameters ) Input Data(POST) | { " ID " : 2 } |  |
| Success Response Data | { " Response " : { " ResponseURL " : "/ HAPI /V1.0/ Event / subscription / delete " " SessionID " : "" , " ResponseCode " : 0 , " ResponseString " : " Succeed " , "Data" : { "ID" : 2 } } } | , |
| Note | Parameter definition see 3.2.1 |  |
| Status |  |  |

HTTP API  Instruction Manual

## 3.3 Event Notification

## 3.3.1 Event Notification / HAPI / V1.0 / Event / Notification

| URL | /HAPI/V1.0/Event/Notification |
| --- | --- |
| Description | Push event notifications. |
| Method | POST |
| Input Data | { "AlarmType":, "AlarmSubType":, "TimeStamp": "ChannelNo": "NotificationType":, "DeviceID":, "Picture":, "AlarmInfo": } |
| Success Return Data | None |
| Note |  |
| Status |  |

| AlarmType | M | string | Alarm Type Ginseng See surface grid 1 AlarmType/ AlarmSubType Item kind type | " ObjectDetect " |  |
| --- | --- | --- | --- | --- | --- |
| AlarmSubType | M | string | Alarm Type Ginseng See surface grid 1 AlarmType/ AlarmSubType Item kind type | " HumanShapeDetect " |  |
| TimeStamp | M | unsigned long | Alarm time. UTC Time, from 1970 Year 1 moon 1st​ 0 The number of seconds from the start of the point. | 1489040894 |  |
| Channel | M | unsigned long | Alarm source channel ID . Single channel product fixed filling 0 ; Multi-channel products 0 Indicates channel-independent things Channel events fill the channel ID Number (from 1 start). | 1 |  |
| NotificationType | M | unsigned long | Notification Type: 0 : Real-time notification 1 : Historical notifications | 0 |  |
| OccurFlag | M | String | Alarm occurrence mark: “ true ”: alarm occurs |  | true |

HTTP API  Instruction Manual

|  |  |  | "false": Alarm ends |  |  |
| --- | --- | --- | --- | --- | --- |
| DeviceID | M | string | Alarm equipment ID , fill in the device serial number, Or other customization required by the alarm server Identifier | " EF 000000000000A3" |  |
| Picture | C | Json Block | jpg Picture information, carry pictures size and Data , Data For jpg conduct base64 Encoded string |  |  |
| AlarmInfo | M | Json Block | According to different AlarmType , carrying different Structured data information, as well as image information Interest |  | - |

sheet 1 AlarmType/AlarmSubType event type

| AlarmType/Event Type | AlarmSubType/Event Subtype | Event Description |
| --- | --- | --- |
| MotiDetect | MotiAlarm | Motion detection alarm |
| ObjectDetect | HumanShapeDetect | Human detection alarm |
|  | VehicleDetect | Vehicle detection alarm |
|  | FallingObjectsDetectiAlarm | High-altitude object throwing alarm |
|  | ElectricbicycleDetect | Motorcycle Inspection |
|  | HumanShapeDetect | Human detection alarm disappears |
|  | VehicleDetect | The vehicle detection alarm disappears |
|  | FallingObjectsDetectiAlarm | The high-altitude object throwing alarm disappeared |
|  | ElectricbicycleDetect | Motorcycle detection alarm disappears |
| MaskDetect | MaskImageAlarm | Occlusion detection alarm |
| AudioDetect | AbnormalAudio | Audio anomaly detection alarm |
| CrosslineDetect | LineDetectorCrossed | Cross-border alarm |
| RegiDetect | EnterArea | Enter the area |
|  | LeaveArea | Leaving the area |
|  | Loitering | Regional wandering |
|  | LoiteringDetecti | Loitering Detection |
|  | Fence Crossing | Climb over the fence |
|  | ObjectRemoved | Item Care |
|  | ObjectLeftBehind | Items left behind |
|  | PeopleGathering | Gathering of people |
|  | AreaPeopleCountingAlarm | Area people counting alarm |
| FireDetect | FireAlarm | Flame alarm |
|  | FumesAlarm | Smoke alarm |
|  | FlameAndFumesAlarm | Fireworks warning |
| FaceDetect | FaceIsDetected | Face Detection |
|  | FaceRecogniti | Face matching alarm |
|  | FaceMatchAlarm | Face recognition matching alarm |
|  | FaceRecognitiMatchlistAlarm | Face recognition matching alarm |

HTTP API  Instruction Manual

|  | FaceNotMatchAlarm | Face recognition mismatch alarm |
| --- | --- | --- |
|  | FaceRecognitiMismatchlistAlarm | Face recognition mismatch alarm |
| LprDetect | LprDetectAlarm | License plate recognition alarm |
|  | LprMatchlistAlarm | License plate recognition matching alarm |
|  | LprMismatchlistAlarm | License plate recognition mismatch alarm |
|  | LprBlacklistAlarm | Parking lot vehicle identification blacklist alarm |
| IllegalLogin | IllegalLogin | Unauthorized access |
| BatteryDetect | LowBattery | Low battery alarm |
| NetworkDetect | NetworkDiscnected | Network disconnection abnormal alarm |
|  | IPCflict | IP Conflict abnormality alarm |
| IOInputDetect | InputAlarm | Input switch alarm |
| TemperatureDetect | TemperatureDetectiAlarm | Temperature detection event |
|  | HighTemperatureHighAlarm | Over temperature alarm |
|  | TemperatureLowAlarm | Low temperature alarm |
|  | TemperatureAbnormalAlarm | Abnormal temperature alarm |
| HumanStatusDetect | SafetyHelmetAlarm | Not wearing a safety helmet |
|  | TelephingAlarm | Call the police |
|  | SmokingAlarm | Smoking warning |
|  | BodyTemperatureAlarm | Abnormal body temperature alarm |
|  | NoMaskAlarm | Not wearing a mask warning |

Example of motion detection alarm message:

POST /HAPI/V1.0/Event/Notification HTTP/1.1 User-Agent: ANJVISION HA PI NOTIFY

Cache-Control: no-cache Connection: keep-alive

Content-Type: application/x-www-form- urlencoded x-cos-meta-User-ID: 1

Host: 192.168.1.253  Content-Length: 983

{"AlarmType":"MotionDetect","AlarmSubType":"MotionAlarm","TimeStamp" :1705310501,"ChannelNo":0,"Notification cationType":0,"OccurFlag":"true","DeviceID":"EF000000000000A3","AlarmInfo":{"image":null,"MotionDetectIn fo ":{"motion_col":22,"motion_row ":18,"motion_details":[{"motion_row":"0000000000000000000000"},{"motion_ row":"0000000000000000000000"},{"motion_row":"000000000000000000000"},{"motion_row":"0000000000  000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"000000000000000000000"},{"  motion_row":"0000000000000000000000"},{"motion_row":"00000000000000000000000"},{" motion_row":"000   00001000000000000000"},{"motion_ row":"000000000000000000000"},{"motion_row":"010000000000000000   0000"},{"motion_row":"00000000000000000000000"},{"motion_row":"0000000000000000000000" },{"motion_row":"0000000000000000000000" },{"motion_row w":"00000000000000000000000"},{"motion_row":"000000000000000000000"},{"motion_row":"0000000000000

0000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"000000000000000000000"} ] }}} Example of motion detection alarm end message:

HTTP API  Instruction Manual

POST /HAPI/V1.0/Event/Notification HTTP/1.1 User-Agent: ANJVISION HA PI NOTIFY

Cache-Control: no-cache Connection: keep-alive

Content-Type: application/x-www-form- urlencoded x-cos-meta-User-ID: 1

Host: 192.168.1.253  Content-Length: 182

{"AlarmType":"MotionDetect","AlarmSubType":"MotionAlarm","TimeStamp" :1705310512,"ChannelNo":0,"Notification cationType":0,"OccurFlag":"false","DeviceID":"EF000000000000A3","AlarmInfo":null}

# 4 JSON structured data description

## 4.1 Arming Mode​

The deployment mode can be configured as: disabled, enabled all day, enabled during the day, enabled at night, and customized time period. The definitions are as follows:

typedef enum {

ARMING_DISABLE = 0, // Disarm

ARMING_ALLDAY = 1, // Arming all day  ARMING_DAYTIME = 2, // Arm during the day ARMING_NIGHT = 3, // Deployment at night

ARMING_CUSTOM = 4, // Custom time period }ArmingMode;

When the deployment mode is a custom time period, the alarm detection deployment configuration triggers various alarm linkages according to the weekly plan ; The defense configuration triggers linkage according to the daily plan.

## 4.2 Alarm detection and arming configuration

Alarm detection arming configuration, different from the alarm detection enabling switch, is mainly used to control all alarm linkage outputs. When enabled, you can enable or disable all alarm linkage outputs according to the deployment plan. When in the system, each alarm linkage control will output according to its own independent deployment plan.

For the definition of alarm detection and arming mode, see 4.1 . When configuring a custom time period, follow 7x24 Hourly configuration of the weekly deployment schedule, minimum time Unit is 1 hours.JSON​ When the data is expressed, the 1 indivual 32 It is expressed as a 1-bit integer , which means the occupancy per hour is 1 indivual bit Come.

JSON data:

" ArmingFlag " : {

" ArmingMode " : 1 ,

HTTP API  Instruction Manual

" WeekPlan " : {

" day 0" : 16777215 , " day 1" : 16777215 , " Day 2" : 16777215 , " day 3" : 16777215 , " Day 4" : 16777215 , " Day 5" : 16777215 , " day 6" : 16777215

} }

## 4.3 Motion detection area configuration

Motion detection area according to the maximum 18 OK twenty two Each sub-area is configured by using characters '0' and '1' to indicate that motion detection in that area is enabled. Measurement.

JSON In the data, the region configuration uses JSON An array of strings.

" AreaConfig " : {

" block_col " :​​ twenty two , " block_row " :​​ 18 , " blockconfig " : [

{

" block_row " :​​ "00011111111111111 11111" },

{

" block_row " :​​ "00011111111111111 11111" },

{

" block_row " :​​ "00011111111111111 11111"

}, {

" block_row " :​​ "00011111111111111 11111" },

{

" block_row " :​​ "11111111111111111 11111"

}, {

" block_row " :​​ "11111111111111111 11111" },

{

" block_row " :​​ "11111111111111111 11111" },

{

" block_row " :​​ "11111111111111111 11111"

HTTP API  Instruction Manual

}, {

" block_row " :​​ "111111111111100000 00000" },

{

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000"

}, {

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000" },

{

" block_row " :​​ "00000000000000000 00000" }

] },

## 4.4 Alarm linkage deployment configuration

For the definition of alarm linkage arming mode, see 4.1 , used to configure linkage daily plan. When configured as a custom time period, configure the maximum 4 indivual

period

JSON data:

{

" ArmingMode ": 4,  " TimeSpanNum ": 1, " DayPlan ": [

HTTP API  Instruction Manual

{

" startTime ": "00:00:00", " endTime ": "23:59:59"

} ]

}

## 4.5 Alarm sound linkage configuration

Alarm tone linkage is used to configure the prompt tone played by the device when an alarm is triggered , as well as the alarm tone linkage deployment plan.

JSON data:

" AudioAction ": {

" times ": 1,

" intervalsecnods ": 10,

" filename ": "/ opt / ch / attention _ en . mp 3", " ArmingSetting ": {

" ArmingMode ": 0,  " TimeSpanNum ": 1,

" DayPlan ": [ {

" startTime ": "00:00:00", " endTime ": "23:59:59"

} ]

} }

## 4.6  IO Output linkage configuration

IO output linkage configuration is used to configure linkage control when alarm is triggered. IO output list. The IO output deployment plan is determined by the IO output configuration. JSON data:

" IOOutputAction ": { " channelCnt ": 4,

" channels ": [ {

" portIndex ": 1, " enable ": 0

}, {

" portIndex ": 2,

HTTP API  Instruction Manual

" enable ": 0 },

{

" portIndex ": 3, " enable ": 0

}, {

" portIndex ": 4, " enable ": 0

} ]

}

## 4.7  Object detection type

The target detection type is used to configure the target type in the target detection algorithm. Currently, human figures and car models are mainly used. JSON data:

" type ": {

" CAR ": 0,  " MOTO ": 0,

" ELECTRICBICYCLE ": 0,

" BICYCLE ": 0, " HUMAN ": 1,

" FACE ": 0,

" NONMOTO_VEHICLE " : 0, " FIRE ": 0,

" FALLINGOBJECT ": 0 }

## 4.8  Target detection area configuration

Target detection area configuration is used to configure the area range of target detection, supporting up to An irregular polygonal area with 10 vertices . Use Screen Percentage x/y Coordinates are used to represent the line segments, and no line segments can cross.

JSON data:

" Polygon ": {

" PointCnt ": 7,

" Points ": [ {

"x": 17,

"y": 11 },

HTTP API  Instruction Manual

{

"x": 86, "y": 13

}, {

"x": 95, "y": 83

}, {

"x": 62, "y": 94

}, {

"x": 16, "y": 90

}, {

"x": 5, "y": 37

}, {

"x": 8,

"y": 14 }

] }

## 4.9 Rectangular detection area configuration

Rectangular detection area configuration is used to configure the range of face detection, upper left vertex position , rectangle width and height, and use screen percentage x/ y   Coordinates are used to represent the

JSON data:

" Area ": {

"x": 0,  "y": 0,  "w": 50, "h": 90

}

## 4.10  Cross-border detection rule configuration

Maximum configuration for out-of-bounds detection 4 Rules . See also 4.7. Each rule configures the coordinate positions of the two endpoints ( AB ) of the line , using the screen percentage.

HTTP API  Instruction Manual

Percentage x/y Direction​  Value range: 0: A<->B 1: A->B 2: A<-B (SIDE A MEANS LEFT OF THE LINE) JSON data:

" Rules ": [ {

" enable ": 1,

" Sensitivity ": 80, " type ": 1,

" direction ": 2,

"x0": 50, "y0": 0,  "x1": 50, "y1": 100

}, {

" enable ": 1,

" Sensitivity ": 50, " type ": 1,

" direction ": 0,

"x0": 30, "y0": 10, "x1": 70, "y1": 90

}, {

" enable ": 0,

" Sensitivity ": 0, " type ": 0,

" direction ": 0,

"x0": 0, "y0": 0, "x1": 0, "y1": 0

}, {

" enable ": 0,

" Sensitivity ": 0, " type ": 0,

" direction ": 0,

"x0": 0, "y0": 0, "x1": 0, "y1": 0

} ],

HTTP API  Instruction Manual

## 4.11 Area Detection Rule Configuration

Maximum configuration for out-of-bounds detection 3 Rules . See also 4.7. . Each rule detection type, detection mode ( STAY: stay, ENTER: enter, LEAVE :

Leave). Mode is STAY When you need to configure s tayseconds (alarm waiting time). JSON data:

" Rules ": [ {

" enable ": 1,

" Sensitivity ": 80,

" type ": {

" CAR ": 1,  " MOTO ": 0,

" ELECTRICBICYCLE ": 0,

" BICYCLE ": 0, " HUMAN ": 1,

" FACE ": 0,

" NONMOTO_VEHICLE " : 0, " FIRE ": 0,

" FALLINGOBJECT ": 0 },

" mode ": " STAY ",  " stayseconds ": 0

}, {

" enable ": 0,

" Sensitivity ": 0, " type ": {

" CAR ": 0,  " MOTO ": 0,

" ELECTRICBICYCLE ": 0,

" BICYCLE ": 0, " HUMAN ": 0,

" FACE ": 0,

" NONMOTO_VEHICLE " : 0, " FIRE ": 0,

" FALLINGOBJECT ": 0 },

" mode ": " STAY ",  " stayseconds ": 0

}, {

" enable ": 0,

" Sensitivity ": 0, " type ": {

HTTP API  Instruction Manual

" CAR ": 0,  " MOTO ": 0,

" ELECTRICBICYCLE ": 0,

" BICYCLE ": 0, " HUMAN ": 0,

" FACE ": 0,

" NONMOTO_VEHICLE " : 0, " FIRE ": 0,

" FALLINGOBJECT ": 0 },

" mode ": " STAY ",  " stayseconds ": 0

} ],

## 4.12 Event Notification Json Block

## 4.12.1   MotionDetectInfo

{

"motion_col": twenty two,   "motion_row": 18,

"motion_details": [ {

"motion_row": "00000000111111111111111"

}, {

"motion_row": "0000000010111111111111"

}, {

"motion_row": "0000000010111111 111111" },

{

"motion_row": "0000111010011111 111111" },

{

"motion_row": "0111110111111111 111111" },

{

"motion_row": "1110011111111111111111"

}, {

"motion_row": "1111110100111111111111"

HTTP API  Instruction Manual

}, {

"motion_row": "0111000000011111 111111" },

{

"motion_row": "11110010111100000000000"

}, {

"motion_row": "00000000000000000000000"

}, {

"motion_row": "00000000000000000000000"

}, {

"motion_row": "0000000000000000 000000" },

{

"motion_row": "00000000000000000000000"

}, {

"motion_row": "00000000000000000000000"

}, {

"motion_row": "00000000000000000000000"

}, {

"motion_row": "00000000000000000000000"

}, {

"motion_row": "0000000000000000 000000" },

{

"motion_row": "0000000000000000 000000" }

] }

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| motion_col | M | unsigned long | Image width divided into columns | twenty two |
| motion_row | M | unsigned long | Image height division lines | 18 |
| motion_details | M | array | Each row of sub-region motion detection results, |  |

HTTP API  Instruction Manual

The character '0' indicates that the area is not detected. to motion, '1' to indicate that the area is detected To sports

## 4.12.1   ObjectDetectInfo

Temporarily empty, to be improved according to the algorithm later